Symbolic Policy Library

David Harrison

University of California, Berkeley

1990

The symbolic policy library is designed to allow programmers to create

and modify symbolic and schematic cells.  The library is designed to

be used in conjunction with direct calls to the Oct database.  The

database calls are used to open and close cells and add optional

annotation through the Oct property and bag mechanisms.  This

description includes a short overview of symbolic policy and complete

descriptions of all symbolic policy library functions.

Symbolic Policy

---------------

The Berkeley symbolic layout style consists of an unrestricted set of

components connected together with non-zero width segments where

connectivity is represented explicitly.  Those readers familiar with

"sticks" style layout may recognize some similarity. There

are several differences between Berkeley Symbolic Layout and

"sticks" style layout: symbolic has no restrictions on the number or

type of primitive components, all wiring in symbolic has user-defined

width, symbolic compaction does no wire expansion or device sizing,

and most symbolic tools are constructed to handle hierarchical

designs.  There are five basic components in this layout style:

instances, terminals, segments, nets, and annotation.  Instances are

physical or symbolic cells that are connected using a collection of

segments.  Terminals designate legal connection points for a cell when

it is used in a higher level cell.  Nets explicitly represent the

connectivity of components.  Annotation is used to add information for

other automated tools which may examine or manipulate the design.

These components are described in more detail in the following

paragraphs.

Symbolic design is hierarchical.  Symbolic cells are made up of lower

level cells.  These lower level cells may be either symbolic or

physical cells.  Thus, the overall design takes the form of a tree (or

hierarchy).  In a symbolic cell, the lower level components are called

instances.  An instance contains two important pieces of information:

where to find the component cell (or _master_ cell) and terminals

(legal connection points for the component).  Instances are similar to

a procedure call; the master specification is like a procedure name

and terminals are like arguments to the procedure.

Symbolic views have _terminals_.  Terminals are objects which

represent legal connection points to the cell when it is instantiated

elsewhere.  These terminals are often called _formal_terminals_.  When

the cell is instantiated in another cell, a copy of these terminals is

made and these terminals are called _actual_terminals_.  All

connections in a symbolic cell are made by creating segments which

connect actual terminals together.  New formal terminals can be

created so that the cell can be placed and connected in higher level

symbolic cells.

Instances in a symbolic cell are connected using _segments_.  Segments

are wires with exactly two endpoints, a user defined width, and an

associated layer.  These segments can be non-manhattan.  However,

several automated tools that read or generate symbolic designs deal

only with manhattan paths.  Thus, a majority of Berkeley symbolic

designs are manhattan.

Often connections between components in a symbolic design will require

changes in direction (jogs) or changes between layers.  Connector

instances are used to represent these changes in direction or layer.

These instances are used to represent both same layer and between

layer ("via") connections.  Thus, every segment in a symbolic design

connects exactly two actual terminals of instances (either component

instances or connector instances).

Interconnectivity is explicitly represented in symbolic cells.  As

connections between actual terminals are made, the connected terminals

and segments are added to objects called _nets_.  For example, if the

circuit connectivity was represented as a netlist, nets would be the

equivalent of nodes.  The presence of nets makes it easy to verify the

connectivity of your cell interactively and for programs like

compactors and simulators to use this connectivity to make their job

easier.

Annotation is an important part of symbolic design.  Annotation allows

programs and hand designers to store information about the design

which can be used later by other programs or designers.  The Oct data

manager provides three kinds of annotation: properties, bags, and

object names.  Properties are free form objects that have a name, a

type, and a value.  These objects can be connected to any other object

in the design.  In symbolic design, properties are often attached to

cells, terminals, nets, and instances.  Bags are named objects that

can contain an arbitrary collection of other objects.  These objects

are often used to build up sets of commonly accessed objects for

efficiency reasons.  In symbolic design for example, a bag named

"INSTANCES" contains all non-connector instances and a bag named

"CONNECTORS" contains all connector instances.  This makes it easy to

distinguish between the two.  The final form of annotation is object

names.  Several Oct objects can be named for reference purposes:

instances, nets, and terminals.  Often designers will name these

objects so that they can be referenced easily after simulation.

All symbolic views have two representations: the cell itself and its

abstraction.  The cell itself describes the cell hierarchically (as a

cell made up of other cells which have been connected together).  The

abstraction of a cell is used to allow tools to deal with the cell

without looking at its detailed definition.  The abstraction is

non-hierarchical and much simpler than the cell definition itself.

Cell abstractions can be automatically generated using the program

_vulcan_ (1CAD).

The current symbolic layout style encourages a bottom-up design

methodology.  First, new primitive cells with formal terminals are

created.  Often, these cells are placed in a standard location so

everyone can use them.  Next, new symbolic cells are generated by

instantiating primitive cells and connecting them together.  New

formal terminals are defined so that these cells can be used in higher

level cells.  Vulcan is used to automatically generate protection

frames for each cell.  Higher level symbolic cells are constructed

from these lower level cells in the same manner.

Schematic policy is an extension of symbolic policy where there are

only two layers ("WIRING" and "SYMBOL"), and segment width is

insignificant.  Thus, all connectors in schematic are of the same

type.  Often, two kinds of connectors are used: one to represent

normal wire connections and one to represent "T-junctions" (also known

as solder dots).  The policy library allows the programmer to support

this policy.

A complete formal description of symbolic and schematic policys can be

found in "Oct Policy", a document that is provided with the Octtools.

The descriptions of functions below assume some familiarity with this

policy.

Area Searching

--------------

In symbolic design, it is often useful to discover what items are near

other items.  The symbolic policy library provides functions to search

areas to meet this need.  These functions are actually wrapper

functions on top of those provided by the region package (see

region.doc for details).  The search is generally quite efficient.

These functions work much like the Oct generation functions.  The

programmer starts a sequence by calling symAreaInit, generates items

in the area using symAreaNext, and finishes the sequence with

symAreaEnd.

typedef struct area_dummy_defn {

int dummy;

} *symAreaGen;

void symAreaInit(octObject *fct, octObjectMask mask, struct octBox *area,

symAreaGen *gen)

Initializes a generation sequence that returns symbolic objects

that intersect the box given by `area'.  Only those objects

whose type is one of those defined in `mask' will be returned.

The following types are supported:

OCT_PATH_MASK:       Symbolic segments

OCT_INSTANCE_MASK:  Symbolic instances and connectors

OCT_TERM_MASK:      Symbolic actual terminals

The objects in a generation sequence are returned by successive

calls to symAreaNext().  A sequence is terminated by symAreaEnd().

int symAreaNext(symAreaGen gen, octObject *obj)

Returns the next object in the area sequence defined

by symAreaInit().  If there are no more objects, the

routine will return zero.  In all cases, after generating

items using symAreaNext(), the user should call symAreaEnd()

to reclaim resources consumed by the generator handle.

void symAreaEnd(symAreaGen gen)

Frees up the resources consumed by the generator `gen'.  This

can be used at any time during the generation sequence.

Connectors

----------

Several functions are provided for constructing and manipulating

symbolic and schematic connectors.  These functions often require as a

parameter a user-defined function for specifying a connector.  This

function is specified by the symConFunc structure:

#define SYM_MAX_MSG	2048

typedef struct sym_con_func_defn {

int (*func)			/* Function to call            */

ARGS((int nLayers, tapLayerListElement *layerList,

octObject *instance, char why[SYM_MAX_MSG],

char *data));

char *data;			/* User data passed to function */

} symConFunc;

When a connector is required, the policy library will call `func'

passing the following parameters to the function:

nLayers

The number of layer descriptions passed in `layerList'.

layerList

A description of each desired connection.  The description is

taken from the technology access package (TAP (3)):

/* defined in tap.h */

typedef struct tapLayerListElement {

octObject layer;

octCoord width;

int direction;

octCoord angle;

} tapLayerListElement;

The `layer' field specifies the connecting layer, the `width'

field describes the width of the connection, the `direction'

field describes the incoming direction of the connection.  It

is specified by `oring' the bits for each desired direction.

The four manhattan directions are indicated by TAP_LEFT,

TAP_TOP, TAP_RIGHT, and TAP_BOTTOM (TAP_HORIZONTAL and

TAP_VERTICAL are defined for bidirectionally symmetric

connectors).  An arbitrary angle is specified by TAP_WEIRD_DIR

and setting the `angle' to the approach angle in tenths of

degrees.

instance

The routine is expected to fill in an instance of a connector

into this field.  The instance should not be instantiated but

should have the master completely specified as well as the

proper transform.  The policy library will fill in the name

and translation before instantiating the connector.

why

If the routine fails to find an appropriate connector, the

routine should return zero and place an appropriate error

message in this buffer.

data

This field is reserved for other data the programmer may want

to provide to the function.  It will be the same as that

specified in the `data' field of the symConFunc structure.

Many routines in the policy library require the programmer to specify

a connector function.  The library provides a default connector

function that uses tapGetConnector to determine an appropriate

connector:

symConFunc *symDefConFunc;

A new connector is created by calling the function symConnector:

void symConnector(octObject *parent, octCoord x, octCoord y,

int ne, tapLayerListElement elems[], symConFunc *confun,

octObject *connector)

This routine makes and places a connector at the location

given by `x,y'.  It finds an appropriate connector by passing

the connection description information given in `ne' and

`elems' to the connector function `confun'.  The connector is

instantiated and returned in `connector'.

Often new connections to existing connectors are made when

manipulating a symbolic or schematic design.  An existing connector

can be upgraded to handle extra connections using symUpgrade:

int symUpgrade(octObject *term, tapLayerListElement *elem,

symConFunc *confun, octObject *new_conn)

This routine examines the connector whose actual terminal is

`term' to see if it can be upgraded to a new connector that

will allow `elem' to connect to it.  If so, it fills in the

instance in `new_conn' and returns a non-zero value.  If not,

it returns zero.  Note that the connector is not actually

placed.  This must be done with symInstance().

One can discover whether an instance or a terminal is a connector

using symContactP:

int symContactP(octObject *obj)

This routine returns a non-zero status if `obj' is a connector

(or a connector terminal).  A quick check of the connectors

bag is made.  If that is inconclusive,  the CELLTYPE property of

the instance master is checked.  This routine only checks

for instances and instance actual terminals.

Interface Generation

--------------------

As mentioned in the introduction, symbolic policy has two

representations: the cell itself and its abstraction.  Often the

policy library has to examine the abstraction of an instance to

extract necessary information.  If a cell does not have an

abstraction, one must be constructed.  Normally, the symbolic policy

package uses the vulcan protection frame generator to generate such

abstractions.  However, the programmer can override this behavior by

providing its own interface generator:

typedef struct sym_gen_func_defn {

int (*gen_func)		/* Generation function */

ARGS((octObject *inFacet, octObject *outFacet, char *data));

char *data;			/* User data           */

} symGenFunc;

void symSetGenFunc(symGenFunc *new, symGenFunc *old)

This routine set the interface generation function for the symbolic

policy library.  The function is called when the package requires

an interface for a cell and none exists.  See symGenFunc for the

form.  `inFacet' is the contents facet used as input.  `outFacet'

is the facet where the interface should be written.  Both facets

should already be open.  `data' is the same as that provided in `new'.

The routine should return zero if it is unsuccessful.

If `old' is non-zero, it is filled with the old function so that

it can be restored at a later time.  The default interface generation

function uses vulcan(1) to produce an interface.

The interface abstraction for an instance can be found using

symInterface:

void symInterface(octObject *inst, octObject *interface)

This routine returns the interface facet of the cell specified

in `inst'.  If no interface is found,  the routine will

attempt to generate one.  If that is not successful,  the

routine will raise a fatal exception error.  The routine uses

the generator specified by symSetGenFunc.  If no generator is

specified, it uses vulcan to produce an interface.

Instances

---------

Generally, the symbolic policy library assumes a certain order of

creation when constructing new symbolic cells.  The library works best

when instances are placed first and then wired together afterward.

This reduces the amount of processing significantly.  The routine

symInstance is used to create new symbolic instances assuming that no

connection maintenance is required:

void symInstance(octObject *fct, octObject *inst)

This routine creates a new symbolic instance.  The instance may

be either a connector or a real instance.  Connectors will be

connected to the connectors bag.  Instances will be connected

to the instances bag.  Jump terminals will be connected to

the jump terminals bag.  No other net maintenance is done (see

symReconnect() for terminal net maintenance).

One can query to determine whether an object is a non-connector

instance or instance terminal using symInstanceP:

int symInstanceP(octObject *obj)

Returns a non-zero status if `obj' is a real instance (or

a real instance terminal).  A quick check of the instances

bag is made.  If that is inconclusive,  it asks whether

the thing is a connector.  If not, it is an instance.

If an instance created using symInstance has overlapping segments or

other instances, one can use symReconnect() to connect it

appropriately to the abutting elements.  However, when instance

terminals overlap, it is possible that they may be connected using

several possible layers.  When this happens, the symbolic policy

library normally chooses the first layer encountered.  The programmer

can override this functionality by specifying an alternate layer

function:

typedef struct sym_lyr_alt_defn {

int (*lyr_alt_func)		/* Alternate implementation function */

ARGS((octObject *t1, octObject *t2, int num_alt, octObject lyrs[],

char *data));

char *data;			/* User data */

} symLyrAltFunc;

void symSetLyrAltFunc(symLyrAltFunc *new, symLyrAltFunc *old)

This routine sets the current layer alternative function

in the symbolic policy library.  This function is called

when two terminals are to be connected automatically but

there are several possible implementation layers for

the connection.  The function is expected to choose

the appropriate implementation layer:

int func(t1, t2, num, lyrs, data)

octObject *t1, *t2;

int num;

octObject lyrs[];

char *data;

The two actual terminals are passed in `t1' and `t2'.  The

list of possible layers is passed in `num', and `lyrs'.

The routine should return a value between 0 and `num'

representing the choice.  A value outside that range

will cause an error.

An instance that is improperly or incorrectly connected due to

overlapping segments or instances can be connected properly using

symReconnect:

void symReconnect(octObject *inst, symConFunc *confun,

symDelItemFunc *del_func)

This routine examines the connections to the terminals of `inst'

for correctness.  If any inappropriate segments are connected

to any of the terminals, the connection will be removed and

the segment will be deleted (if it is not connected to anything

else).  Appropriate segments that overlap instance terminals

will be connected to the instance.  If an appropriate terminal

of another instance intersects a terminal, a segment between

them will be created.  In general, this routine is used to

cause connectivity updates to an instance and insure proper

connectivity.  It must be used carefully since it is very

aggressive at removing bad connections.  Whenever a bad

connection is removed, the routine calls `del_func'.  The form

of this function is described below under symDeleteObjects().

Often it is useful to replace a symbolic instance with a new similar

instance.  This can be done using symReplaceInst:

void symReplaceInst(octObject *fct, octObject *old, octObject *new)

This routine replaces the instance `old' with `new'.  The

instance `new' will be created and all connections from

`old' will be moved to `new'.  The old instance will then

be deleted.  Actual terminal correspondence is done first

by name then by position.  The routine will raise errors

and print warnings if there are problems.

Segments

--------

Creating symbolic segments implies the creation of nets.  These nets

represent the connectivity between the actual terminals of real

instances.  As segments are created and deleted, appropriate nets are

created.  When two segments are joined together or a segment is used

to connect two instances, the two corresponding nets must be merged

into one.  One of these nets is then destroyed.  The symbolic policy

library uses hueristics to determine which net to save and which to

delete based on net names.  However, there are situations where it

cannot determine which net to preserve.  Under these conditions, it

normally chooses which net to preserve randomly.  It is possible for

the programmer to override this functionality by specifying a net

merge function:

typedef struct sym_merge_func_defn {

int (*func)			/* Function to call */

ARGS((octObject *net1, octObject *net2, char *data));

char *data;			/* User data        */

} symMergeFunc;

void symSetMergeFunc(symMergeFunc *new, symMergeFunc *old)

This routine sets the merge function for the symbolic policy

library.  This function is called only when the package cannot

determine which of two nets should be preserved.  The function

has the following form:

int func(net1, net2, data)

octObject *net1, *net2;

char *data;

If the function returns a non-zero value, net2 will be merged

into net1.  If the function returns zero, net1 will be merged

into net2.  If `old' is non-zero, the old function will be

returned so that the old behavior can be restored if desired.

One can determine the net associated with a symbolic object using

symLocNet:

int symLocNet(octObject *obj, octObject *net)

This routine attempts to find the net for the object `obj'.

The object is expected to be either a path or a terminal.

Returns zero if there is no net associated with the object.

A major part of the heuristics for determining what net to preserve

when nets are merged is based on net naming.  When the policy library

creates new nets, they unnamed.  These are known as machine named

nets.  Nets explicitly named are known as human named nets.  The

package attempts to preserve user named nets over machine named nets.

One can determine whether a net is human named using symHumanNamedNetP:

int symHumanNamedNetP(octObject *net)

Returns a non-zero status if the net is not machine named.

In symbolic designs, all connected terminals are generally physically

connected using segments.  In schematic policy, it is possible to have

terminals that are connected conceptually but are not connected using

wires.  These terminals are used to implement off-page connectors or

supply or ground symbols.  These terminals are designated by a

property on the interface master of the corresponding instance.  They

are known as "jump terminals".  When one creates a segment connected

to a jump terminal, the net is automatically merged with a net whose

name is determined by the value of the the jump terminal property.  If

this property has a null value, it indicates that the user should be

queried for a net name.  The symbolic policy library calls a user

function when this situation occurs:

typedef struct sym_jumpname_defn {

void (*func)		/* Function to call */

ARGS((octObject *term, char jump_name[SYM_MAX_MSG], char *data));

char *data;			/* User data passed to function */

} symJumpNameFunc;

void symSetJumpNameFunc(symJumpNameFunc *new, symJumpNameFunc *old)

This routine sets the global net naming function to `new'

and returns the old function in `old' (for later restoration

if desired).  The form of the function is:

void func(term, name, data)

octObject *term;

char name[SYM_MAX_MSG];

char *data;

This function is called when the package needs to have a

unique name for a global net created according to jump

terminal policy.  `term' is the jump terminal involved.

The routine should write the name in `name'.  `data'

is the same as `data' in `new'.

If the programmer does not provide a jump name function, the library

will automatically name such nets ";;; Global Net ;;;".

New symbolic segments are created using the function symSegments:

void symSegments(octObject *facet, octObject *first, octObject *last,

int np, struct octPoint *points,

int nl, octObject layers[],

int nw, octCoord widths[],

symConFunc *confun)

This routine creates new symbolic segments in `facet' that connect

the terminal `first' with the terminal `last'.  All segments

are implemented as two-point paths in Oct.  The points for these

paths are derived from the array `points' (which must have `np' points,

where np > 2).  The first and last points must intersect the

implementations of `first' and `last' respectively (this is checked).

The layers for the segments are determined by the array

`layers' (which must have `nl' layers, where `nl' > 0).  If

there are fewer layers than segments, the routine will iterate

through the layers to create the segments.  Thus, if ten

segments and two layers are provided, the segment layers will

alternate between the two layers.  The widths for the layers

are determined by the array `widths'.  If there are fewer

widths than segments, the widths are filled using the same

algorithm as that used for layers.  `confun' is a function

that should return an instance of a connector given

information about joining segments.  It is fully described

above under Contacts.  One can use the default connector

function by specifying symDefConFunc as the connector function.

Implementation notes:

1. Internal points are handled by automatically generating connectors

and connecting the adjoining paths to it.  No outside connections

are made at internal points.

2. Net maintenance is done carefully at the end-points to preserve

nets other than the one(s) automatically created by segment

creation.

One can change the size or layer of an existing symbolic segment using

the function symSegChange:

void symSegChange(octObject *fct, octObject *seg, octObject *new_lyr,

octCoord new_width, symConFunc *confun)

This routine changes the layer of a segment to `new_lyr'

and its width to `new_width'.  The routine examines the

connectors at the endpoints of the segment and changes

them as appropriate.  One can specify the default connector

function by specifying symDefConFunc.

One can generate a description of a segment connection for use by the

connector generation functions by using symSegDesc:

void symSegDesc(octObject *seg, struct octPoint *ref,

tapLayerListElement *elem)

This routine returns a description for `seg' suitable for

use with connector functions.  If `ref' is non-zero, it

will be used as the reference point for comparison (allowing

the routine to return more specific directions).  At the moment,

it returns manhattan angle information only and the reference

point is unused.

The function symSegConnP can be used to determine whether a segment

can connect to an existing terminal:

int symSegConnP(octObject *term, tapLayerListElement *elem)

Returns a non-zero value if `term' has an implementation of the

right size on the right layer to accept a connection to the

segment described by `elem'.  Does not consider whether the

terminal could be upgraded.  Use symUpgrade() if that might

be the case.

Terminals

---------

Information about the implementations of actual terminals in symbolic

cells can be found by using the following functions.  This functions

implement a generation style interface like that provided by Oct to

determine both the layers and the implementation boxes for actual

terminals in symbolic cells.

typedef struct term_dummy_defn {

int dummy;

} term_dummy;

typedef term_dummy *symTermGen;

void symInitTerm(octObject *a_term, symTermGen *t_gen)

Starts a generation sequence for returning the implementations

of an actual terminal by examining the interface of the

instance master.  The routine will raise error conditions

if there are errors.

int symNextTerm(symTermGen gen, octObject *lyr, octObject *geo)

This routine generates the next implementation geometry

found for the actual terminal specified in symInitTerm().

If non-zero,  `lyr' will be filled with the implementation

layer for `geo'.  `geo', which must be non-zero,  will be filled with

an implementation geometry properly transformed into the

instance coordinate system.  Returns zero if there

are no more geometries to generate.

void symEndTerm(symTermGen gen)

Releases all resources associated with `gen'.  This should

be done after all generation sequences.

Formal terminals in symbolic and schematic cells are created using the

function symNewFormal:

typedef struct sym_term_func_defn {

void (*func)		/* Function to call */

ARGS((octObject *fterm, octObject *obj, int detach, char *data));

char *data;			/* User data        */

} symTermFunc;

void symNewFormal(octObject *facet, octObject *fterm, octObject *implBag,

char *termtype, char *termdir, symTermFunc *term_func)

This routine creates a new symbolic formal terminal `fterm' whose

implementation is all of the actual terminals and segments in

`implBag'.  TERMTYPE and  TERMDIRECTION properties will be added

to the terminal with the values provided in `termtype' and `termdir'.

The caller should delete the implementation bag if there is no

further use for it.  Any previous implementations for the terminal

are removed.  If supplied, `term_func' will be called

when items are attached or detached from the new terminal.  The

form of the function is given above in the declaration for

symTermFunc.  `fterm' will be the terminal, `obj' will be the

item that is about to be attached or detached from the

terminal, and `detach' will be non-zero if the item is going

to be detached (otherwise attached).

The function symGetTermProp is a convenience function for determining

the annotation on formal terminals:

#define SYM_TERM_TYPE	"TERMTYPE"

#define SYM_TERM_DIR	"DIRECTION"

int symGetTermProp(octObject *term, octObject *prop)

This routine tries to find the property `prop' on the

terminal `term'.  If it fails and the terminal is

an actual terminal, it will continue the search on

the corresponding terminal on the interface master.

If the property is found, it is returned in `prop'

and the routine returns a non-zero status.  If the

property can't be found, the routine returns zero.

Moving, Copying, and Deleting

-----------------------------

Symbolic instances and segments can be moved, copied, and deleted

using the functions described below.  The net maintenance required for

a group of items is often much less than performing the maintenance

required for each item individually.  Thus, the routines below are all

designed to operate on groups of items.  These groups are specified

using Oct bags.

A group of items can be moved using symMoveObjects:

typedef struct sym_move_item_func_defn {

void (*func)		/* Function to call */

ARGS((octObject *item, int after_p, char *data));

char *data;			/* Extra data       */

} symMoveItemFunc;

void symMoveObjects(octObject *set, struct octTransform *transform,

symMoveItemFunc *move_func)

This routine moves the objects in `set' to a new location

computed by applying `transformation' to all objects in

the set.  This routine does NOT change connectivity.  Instead,

segment endpoints are moved with the instances.  `move_func'

is a function that is called when the actual move of an object

is performed. The function will be called twice consecutively

for each item (once before the move and once after).  See

symMoveItemFunc for the argument form.  `item' will be item

moved.  `after_p' will be non-zero if this is the call after

the move.  `data' is the same as passed to this function.

Implementation notes:  the routine builds a set of pure

instances (and other geometry) to move.  Only the instances

are actually moved.  Any segments connected to instance actual

terminals are moved with the instances.  It is possible for

this routine to produce wild symbolic layout so caution is

recommended.

Objects can be moved using the symCopyObjects function:

typedef struct sym_copy_item_defn {

void (*func)		/* Function to call */

ARGS((octObject *old, octObject *new, char *data));

char *data;			/* User supplied data */

} symCopyItemFunc;

void symCopyObjects(octObject *facet, octObject *set,

struct octTransform *transform,

octObject *result,

symConFunc *confun, symCopyItemFunc *ci)

This routine copies the objects given by `set' to a new location

that is computed by applying `transform' to all objects in

the set.  The new copies of the objects are attached to the

supplied bag `result'.  Connectivity is maintained.  Restriction:

no connects will be made to any objects already in the destination

area.  Use symReconnect() if this behavior is required.

`ci' is a function called when the actual copy of an object

is performed.  The prototype is shown above in the definition

of symCopyItemFunc. `old' will be the old object, `new' will

be the newly copied object, and `data' will be the `data'

supplied to this function.  If the function is not supplied,

no callback will be done.  New connector objects may be added

to the copy set.  These will be denoted by having `old' be

zero.

A set of objects may be deleted using symDeleteObjects:

typedef struct sym_del_item_func_defn {

void (*func)		/* function to call */

ARGS((octObject *item, char *data));

char *data;			/* User data        */

} symDelItemFunc;

void symDeleteObjects(octObject *delSetBag, symDelItemFunc *del_func)

Deletes all items in `delSetBag'.  `del_func' specifies a function

that is called just before each item in the set is deleted.

See the definition of `symDelItemFunc' for the form.  `item'

will be the object that is to be deleted and `data' will be

the user data in `del_func'.  If no del_func is supplied,

no callback is made.

Error Handling

--------------

The symbolic policy library may produce information messages,

warnings, and errors.  Truely fatal errors are signaled using errRaise

(see the documentation on errTrap for details).  These errors are

reported by calling a warning function.  The default warning function

writes messages to stderr and continues.  The programmer can override

this behavior by providing a user defined function:

typedef enum sym_severity_defn { SYM_INFO, SYM_WARN, SYM_ERR } symSeverity;

typedef struct sym_warn_func_defn {

void (*func)		/* Function to call */

ARGS((symSeverity severity, char *message, char *data));

char *data;			/* User data        */

} symWarnFunc;

void symSetWarnFunc(symWarnFunc *new, symWarnFunc *old)

This routine sets the warning function for the symbolic policy

library.  This function is called when a non-fatal error is

detected or a questionable condition arises.  Fatal errors

are signalled using the errTrap package.  The function

has the following form:

void func(severity, message, data)

symSeverity severity;

char *message;

char *data;

The parameter `severity' will indicate the severity of the error.

`message' will contain text describing the error or warning.

`data' is the same as that provided by the user.  If `old' is

non-zero, the old function is returned so that it can be reinstated

at some later time if desired.  The default function prints the

messages to standard error.

As mentioned above, fatal errors are signalled with errTrap.  The

symbolic policy library defines the following fatal error types:

#define SYM_OCTFAIL	0	/* Oct call failed              */

#define SYM_NONET	1	/* Object has no associated net */

#define SYM_MAPFAIL	2	/* Group map table failure      */

#define SYM_BADOBJ	3	/* Bad object type              */

#define SYM_BADTERM	4	/* Formal/actual mismatch       */

#define SYM_NOINTR	5	/* No interface facet           */

#define SYM_NOMASTER	6	/* No master for instance       */

#define SYM_PERM	7	/* Permission failure           */

#define SYM_BADPOL	8	/* Cell has bad symbolic policy */

#define SYM_MULNET	9	/* Too many nets                */

#define SYM_BADBAG	10	/* Bad policy bag maintenance   */

#define SYM_BADSEG	11	/* Bad symbolic segment         */

#define SYM_BADFUNC	12	/* Bad user function            */