network_t *

network_alloc()

Allocate and initialize a new network.

void

network_free(network)

network_t *network;

Free a network.

network_t *

network_dup(network)

network_t *network;

Make a duplicate copy of a network.

network_t *

network_create_from_node(node)

node_t *node;

Create a new network with a single internal node with the same logic

function as 'node'.  Primary inputs will be created for this

new network with the same names as the fanins of 'node'.

char *

network_name(network)

network_t *network;

Returns the name of the network.  Do not modify or free the returned

string.

void

network_set_name(network, name)

network_t *network;

char *name;

Sets the name of the network.  'name' is copied.

int

network_num_pi(network)

network_t *network;

Return the number of primary inputs for the network.

int

network_num_po(network)

network_t *network;

Return the number of primary outputs for the network.

int

network_num_internal(network)

network_t *network;

Return the number of internal nodes in the network.

node_t *

network_get_pi(network, index)

network_t *network;

int index;

Return the index'th primary input of the network.  The primary

inputs are numbered starting from 0.  Returns NIL(node_t) if

index >= network_num_pi(network).  Potentially a very slow way

to access an arbitrary primary input.

node_t *

network_get_po(network, index)

network_t *network;

int index;

Return the index'th primary output of the network.  The primary

outputs are numbered starting from 0.  Returns NIL(node_t) if

index >= network_num_po(network).  Potentially a very slow way

to access an arbitrary primary output.

int

network_check(network)

network_t *network;

Performs various consistency checks on a network.  Used for

debugging the network package.  If the network is constructed

using only the routines in this package, it should never fail

this check.  Returns 1 if the network is okay, or 0 if the

network contains some internal inconsistency.  In the case of

error, error_string() (see error.doc) can be used to retrieve

a description of the error.

int

network_is_acyclic(network)

network_t *network;

Checks that the network does not contain a cycle.  Returns 1

if there are no cycles, or 0 if there is a cycle.  In the case

of a cycle, error_string() (see error.doc) can be used to

retrieve a description of the nodes on the first cycle

encountered.

/* Generators */

foreach_node(network, gen, node)

network_t *network;

lsGen gen;

node_t *node;

Generates over the all nodes in a network (that is, including

primary inputs and primary outputs).

foreach_primary_input(network, gen, pi)

network_t *network;

lsGen gen;

node_t *pi;

Generates over the primary inputs of a network.

foreach_primary_output(network, gen, po)

network_t *network;

lsGen gen;

node_t *po;

Generates over the primary outputs of a network.

The generators are macro's.  Typical usage is:

foreach_node(network, gen, node) {

/* do something with node */

}

If the loop is terminated abnormally (i.e., with a break or return),

lsFinish() must be called on the generator.

In general, the generator operation is not well-defined if any attempt

is made to add a node to the network during the generation loop.

void

network_add_node(network, node)

network_t *network;

node_t *node;

Adds a node to the network.  The node will be assigned

automatically generated names (both long and short) if it is

currently unnamed.  It is a serious error if the node has the

same name (or same short_name) as any node already in the

network.

void

network_add_primary_input(network, node)

network_t *network;

node_t *node;

Adds a primary input node to the network.  The node will be assigned

automatically generated names (both long and short) if it is

currently unnamed.  It is a serious error if the node has the

same name (or same short_name) as any node already in the

network.

node_t *

network_add_primary_output(network, node)

network_t *network;

node_t *node;

Designate the node 'node' as a primary output node.  Creates a

new primary output (of type PRIMARY_OUTPUT) node which is returned.

Node must be a node already in the network.

void

network_delete_node(network, node)

network_t *network;

node_t *node;

Deletes a node from the network.  The behavior is undefined if

this routine is called from within a generator.  It is a

serious error to attempt to delete a node which is not already

in the given network.

void

network_delete_node_gen(network, gen)

network_t *network;

lsGen gen;

Deletes the last node generated by the generator gen.  Behavior

is well-defined when called from within a generator loop.  It

is a serious error to attempt to delete a node which is not

already in the given network.

node_t *

network_find_node(network, name)

network_t *network;

char *name;

Find the node in the network with the given name.  Note that

the behavior of this routine changes depending on the

name_mode variable (i.e., whether to look at the long names,

or the short names).

void

network_change_node_type(network, node, new_type)

network_t *network;

node_t *node;

node_type_t new_type;		/* see node.doc for definition */

Change the type of a node.  In particular, this can be

used to designate certain nodes as either primary inputs

or primary outputs.

void

network_change_node_name(network, new_name)

network_t *network;

node_t *node;

char *new_name;

Change the name of a node.  new_name is taken directly; you may

wish to use strsav() to make a duplicate copy which is passed

to network_change_node_name.

int

network_collapse(network)

network_t *network;

Collapse a network into one level of nodes (i.e., two-level).

Returns 1 to indicate if the network changes, 0 otherwise.

int

network_sweep(network)

network_t *network;

Propogate (and eliminate) constant-valued nodes in the network.

Also deletes inverter and buffer nodes, and nodes which do not

fanout anywhere.  Returns 1 if the network is changed, or

returns 0 if not.

int

network_cleanup(network)

network_t *network;

Removes internal nodes which do not fanout anywhere.  Returns 1

if the network is changed, or returns 0 if not.

array_t *

network_dfs(network)

network_t *network;

Returns a vector of nodes ordered in a depth-first manner from the

outputs.  Includes PRIMARY_INPUT, PRIMARY_OUTPUT and INTERNAL nodes.

(The nodes are ordered such that every node appears somewhere after

all of its transitive fanin nodes.)

array_t *

network_dfs_from_input(network)

network_t *network;

Returns a vector of nodes ordered in a depth-first manner from the

inputs.  Includes PRIMARY_INPUT, PRIMARY_OUTPUT and INTERNAL nodes.

(The nodes are ordered such that every node appears somewhere after

all of its transitive fanout nodes.)

array_t *

network_tfi(node, level)

node_t *node;

int level;

Returns a vector of nodes ordered in a depth-first search towards

the primary inputs from a given node.  Nodes up to a maximum depth

of level are returned.  (Level 0 is no nodes, level 1 corresponds

to the immediate fanin nodes, etc.)  The list includes PRIMARY_OUTPUT

nodes as appropriate.  The list does NOT include the node `node' itself.

array_t *

network_tfo(node, level)

node_t *node;

int level;

Returns a vector of nodes ordered in a depth-first search towards

the primary outputs from a given node.  Nodes up to a maximum depth

of level are returned.  (Level 0 is no nodes, level 1 corresponds

to the immediate fanout nodes, etc.)  The list includes PRIMARY_INPUT

nodes as appropriate.  The list does NOT include the node `node' itself.

pPLA

network_to_pla(network)

network_t *network;

Returns the network as an Espresso PLA structure.  A copy

is made of the network, this is collapsed to two-levels, and

then mapped into the Espresso multiple-output PLA format.

The original network is unaffected.  The cube_size (that

nasty Espresso global variable) is set to reflect the PLA

that is returned.  No other node/network operations may be

performed until discard_pla() is called.

pPLA

espresso_read_pla(fp)

FILE *fp;

Uses the espresso PLA read routine to read a PLA from a file.

Returns NIL(PLA_t) if there is any error reading the PLA.

The cube_size (that nasty Espresso global variable) is set

to reflect the PLA that is returned.  No other node/network

operations may be performed until discard_pla() is called.

void

discard_pla(PLA)

pPLA PLA;

After using network_to_pla(), or espresso_read_pla(), normal

node/network operations CANNOT be performed until the PLA

is discarded using this routine.  Free's the PLA, and restores

the `cube' for normal MIS operation.

network_t *

pla_to_network(PLA)

pPLA PLA;

Maps a PLA into a network.  This creates a two-level structure

of NOR-NOR gates (surrounded by inverters to get the phase

correct).  That is, each node in the resulting network will be

either an inverter, or a NOR gate.  Assumes the Espresso global

variable `cube' accurately describes the layout of the PLA.

(This will be the case if the PLA is created using

network_to_pla(), or espresso_read_pla().)  If the PLA is not a

multiple-output PLA (i.e., if it contains any multiple-valued

variables), then NIL(network_t) is returned.

network_t *

network_espresso(network)

network_t *network;

Runs espresso on a network and returns the optimized network.

Uses pla_to_network() to create a NOR-NOR network.  The original

network is unaffected.

void

network_reset_long_name(network)

network_t *network;

Re-create unique names for all nodes which have 'made-up' names.

(i.e., a name of the form [%d]).  Also renumbers them starting

from 1.

void

network_reset_short_name(network)

network_t *network;

Re-create short names for all nodes.  Renumbers the short names

starting with a.  Primary inputs and primary outputs are

assigned the lowest valued names, followed by internal nodes.