Bus Policy

Level zero conformance

David Harrison

Bus:	A possibly heirarchical collection of wires. Each bus may

contain a number of other busses.

Levels:	Bus policy is by nature complex.  Several applications have

been identified each requiring differing levels of bus

abstraction.  Implementing the most general and complex bus

abstraction policy may take an unacceptably long period of

time (due to the number of tools impacted and the complexity

of the policy itself).  Thus, bus policy has been broken into

levels of conformance.  This document describes level zero

conformance.  Level zero represents busses using a concrete

representation with explicit widths and types.  Higher level

policies are under consideration for behavioural design.

Base Policy:

Zero level bus policy is a strict extension of symbolic

policy.	Connectivity is represented using nets attached to

segments that implement the nets.  All segments are two-point

paths. Segments are attached to the actual terminals of their

end-point connectors.  Actual terminals of non-connector

instances are attached to exactly one net.  Actual terminals

of connector instances are not attached to a net.

Global Policy:

All cells with bus representations must use the upward

compatible version of Oct that allows net and terminal objects

to have a settable width (Oct 2.0).

Cells with bus representations shall have an integer valued

property named "BUS_LEVEL" attached to the facet.  The value

of this property will be the level of conformance of the cell

(for this policy,  the value would be zero). Facets without

this property will be guaranteed to have no busses.

Net Policy:

Singular nets are represented in the same way as nets in

symbolic policy except the width field in the net object is

explicitly set to zero.  This is a change in policy from

symbolic policy where the implied with of nets and terminals

was one.  However, as shown below, a net with a width of one

may be considered an indivisible bus.  Since current symbolic

policy differs from this policy, netlisting tools will have to

examine the BUS_LEVEL property to determine singularity of

nets.  Presence of a BUS_LEVEL property means nets are

singular iff the width field of the net object is zero.

Absence of the BUS_LEVEL property means all nets are singular

regardless of the width field of net objects.

Bus nets are represented as net object with a width greater

than zero.  The meaning of the width is defined as the number

of sub-components of the bus.  The number of sub-components is

not the same as the number of wires.  Each sub-component may

itself consist of other sub-components, forming a heirarchy.

At any one level of the hierarchy,  the width represents the

number of "entities" that can be "ripped" off of the bus (see

Ripper Policy below).  Thus, the tree structure of the

bus hierarchy is derived by examining the width fields of nets

as they pass through instances and connectors.

Connectivity information for bus nets is similar to that

specified for symbolic policy.  However, under symbolic policy

it is always possible to merge any two nets into one net.

The implicit nature of the bus hierarchy makes this

problematic in a bus cell.  Obviously, nets with different

widths cannot be merged directly.  Furthermore, two nets with

the same width may not be compatible.  One net may represent a

bus of three wires whereas the other may represent a bus of

three busses each with more than one wire.  We solve this by

introducing a new annotation to nets: a BUSTYPE property.  The

BUSTYPE property is a string valued property specifying a name

for the bus type.  The mergability of nets can be expressed as

follows:

Two nets can be merged into one net if they are

compatible.  All nets with widths of zero are

compatible with each other.  Two nets with widths

greater than zero are compatible iff they have the

same width and they share the same BUSTYPE.  A net

without a BUSTYPE property is considered incompatible

with all other nets.

Intuitively, the BUSTYPE property represents a user-defined

name given to a certain class of bus.  Typical BUSTYPE

annotations might be "OPCODE" or "ADDRESS".  Generally,  these

names should be unique in each cell but don't have to be

unique across an entire design.

Terminal Policy:

Like nets, singular terminals are represented as terminals

with a width explicitly set to zero.  Singular terminals are

handled in the same fashion as in symbolic/schematic policy.

Composite terminals are represented as a terminal object with

a width greater than zero.  The meaning of the width is the

same as that defined for nets.

Connectivity concerns for terminals are also similar to those

described for nets.  Terminals also must have a BUSTYPE

specifying a name for the type of bus that can connect to the

terminal.  However, the BUSTYPE attached to an actual terminal

may differ from that specified on the corresponding formal

terminal.  This underlines the policy described above that

BUSTYPES may differ at different levels of the instance

hierarchy.

Generally, nets may only be connected to terminals that are

compatible with the net.  Compatibility for terminals and nets

is similar to compatibility between nets:

All nets and terminals with a width of zero are

compatible with each other.  Nets and terminals with a

width greater than zero are compatible iff they have

the same width and they share the same BUSTYPE.

There are some important implementation notes to add to

terminal bus policy.  Terminals in a cell may be formal

terminals (exported for use when the cell is instantiated) or

actual terminals (those used to connect to instances in the

cell).  Terminal widths can be modified only by modifying

formal terminals.  Thus to change the width of an actual

terminal, one must open the "contents" facet of the master of

the corresponding instance and modify the width in the

instance.  This means modifying masters will be a more

frequent operation.

Connector Policy:

In symbolic/schematic policy, special instances called

connectors are used to join segments that implement a net.

Generally, these instances are chosen automatically from a

predefined set of connectors available through a technology

library.  In level zero bus policy, this mechanism is still

valid for nets with widths of zero.

Nets with widths greater than zero now require connectors with

terminals that share the same width and BUSTYPE as the net

segments they connect.  Since the width of a terminal may only

be changed in the master, the choice of an appropriate

connector in the technology library must be expanded to

include width.  Since BUSTYPE is an annotation on the actual

terminal, we allow connectors to automatically inherit the

BUSTYPE of the net they connect.  Otherwise,  bus connectors

are treated the same way as in symbolic/schematic policy.

Ripper Policy:

Rippers are instances of cells designed to extract portions of

a bus into its component parts.  Intuitively, they can be

considered to be a special kind of connector (however, the

policy for these cells is quite different).

A new value for the CELLTYPE property, "RIPPER", is used to

indicate that a cell is a ripper.  A ripper cell has exactly

two terminals.  In the master of the ripper cell, these

terminals have a width.  These widths represent the width of

the incoming bus and the width of the outgoing bus.  These

widths must match the widths of the bus nets connected to the

actual terminals of the cell.

All instances of ripper cells are attached to a newly defined

bag called "RIPPERS".  Like the "CONNECTORS" bag, this bag is

attached to the facet and allows utilities to quickly

determine what instances are rippers.

The actual terminals of	a ripper cell must be annotated with

the BUSTYPE property.  In addition, exactly one of the two

actual terminals on a ripper cell must have a property named

"BUSOFFSET".  This is an integer valued property that

indicates the extracted component of the other composite net.

This means rippers can only extract one component of a bus.

There is no net maintainence across ripper cells.  In other

words, no relationship is established between a composite net

and its extracted component.  These nets are considered

entirely independent in level zero bus policy.

As an example, suppose a designer wishes to extract the second

component of an eight component bus where this second

component is actually a three component bus.  The ripper cell

used for this would have two terminals: one with width 8 (the

width of the incoming bus) and one with width 3 (the width of

the outgoing bus).  When this ripper is placed in the cell,

the incoming eight component bus is attached to the actual

terminal of width eight and the outgoing three component bus

is attached to the actual terminal of width three.  A BUSTYPE

property is attached to both actual terminals with values that

match the BUSTYPE of the corresponding nets.  Finally, to

indicate that the ripper extracts the second component, a

BUSOFFSET property is attached to the actual terminal of the

outgoing net with a value of one (the first item in the bus is

labeled zero by convention).  Other graphic examples are

attached to this proposal.

Rationale:

There were several alternatives suggested for representing

busses.  Many of these ideas were valid but were rejected in

favor of a simple first level policy that could be implemented

reasonably quickly.  Some of these alternatives are presented

below.

There is a spectrum of possible represetations for composite

nets.  On the simple end of the spectrum, busses can be

represented by a fixed width collection of wires (no

hierarchy).  This is by far the easiest to implement but does

not allow representations that are common in modern digital

design.  On the other end of the spectrum, busses can be

represented using abstract types (with no width specification

at all).  The structure of this type can only be derived by

examining the entire design using some reasonably complex

bookkeeping techniques.  The level zero bus policy chooses a

point between these two extremes where hierarchy of busses is

allowed but the number of components at each level is

explicitly represented using widths.  The widths allow some

rudementary checking to be implemented and a user-maintained

BUSTYPE is used to allow tools to quickly determine bus

compatibility.  Higher level bus policy may indeed allow

BUSTYPE to be extended so that widths are insignificant.

The proposed terminal policy also has advantages and

disadvantages.  Because the terminal width is represented as a

field in the terminal, widths are not easy to change (for a

given actual terminal, the corresponding formal terminal of

the master of the contents facet must be modified).  Early

proposals suggested the terminal width be an annotation rather

than an addition to the terminal object.  However, by placing

the width in the terminal,  the width is guaranteed to be

consistent in all corresponding actual terminals and formal

terminals of other facets.  Futhermore, changing the width of

a bus in level zero policy is a major operation that requires

touching a great deal of the design.  The additional overhead

of opening the masters of instances was considered

inconsequential.  The only major disadvantage of this policy

concerns ripper cells.  Unique ripper cells are required for

each width combination of the incoming and outgoing composite

net.  Since the number of combinations could be large,  means

for automatically generating ripper cells must be provided.

Rippers are by far the most complex component of the level

zero bus policy.  The proposed policy follows the semantics of

ripper cells in many industrial schematic entry systems.  The

ripper is allowed to pull off only one component of the

composite net and this is specified by an integer offset.  The

addition made in this policy is allowing the outgoing net to

have a non-zero width thus implying hierarchical bus

structures.  Other policies were suggested that allowed

greater control over the interconnection between busses.

However, after some thought, the proposed policy was found to

be sufficient to cover all of the cases as the more complex

switchbox ripper cells but is substantially simpler to build

and maintain.  If warranted, later revisions of the policy may

allow rippers with more than two terminals as a shorthand

device.

The most controversial aspect of the policy is the choice not

to maintain any relationship between composite nets and

subnets ripped off through ripper cells.  Early proposals

specified that subnets of a composite net be attached to the

primary net.  This would allow interactive tools to highlight

bus nets in an intuitive fashion.  However,  the maintenence

required to maintain this connectivity information is

extensive.  It was decided that level zero policy will not

require this maintenence.  Later policies may attempt to

resurrect this level of connectivity information.

Examples: