Array assembly package

Chuck Kring   ( kring@ic.Berkleley.EDU )

University of California, Berkeley

------------------------------------------------------------------

This is a package of functions which are useful for creating array

based layouts in OCT.

The header file is: 	$OCTTOOLS/include/mkarray.h

The library is:		$OCTTOOLS/lib/libmkarray.a

The lint library is:	$OCTTOOLS/lib/llib-lmkarray.ln

------------------------------------------------------------------

Routines to manage arrays:

Arrays are the underlying framework presented by mkarray.  Once

an array has been opened and sized, instances and terminals can

be attached to the array and the array can be used to specify

regions for pattern based wiring.  After the layout in the

array has been specified, a constraint graph which models the

placement of the array is derived and solved.  The translation

of the instances in the array is recovered from the graph and

the interconnect in the array is created.

void

mkOpenArray( array, container )

mkId *array;

octObject *container;

mkOpenArray() opens an array which is attached to container.

Container should be either an OCT_FACET or an OCT_BAG.  If

the container is an OCT_FACET, a bag named "MKARRAY" will be

attached to the facet and will contain any error or debugging

bags which are created by mkarray.  Otherwise, any error or

debugging bags are attached directly to the container.

An allocated mkId must be passed into mkOpenArray() and will

be set to a number which identifies the array.  This id is used

in subsequent calls to mkarray which operate on the array.

mkOpenArray() returns MK_OK, MK_BAD_CONTAINER, or MK_OCT_ERROR,

depending upon whether everything was ok, the container was not

an OCT_FACET or an OCT_BAG, or and error was detected in an OCT

call.

void

mkAddArrayArea( array, area )

mkId array;

octBox *area;

mkAddArrayArea() adds area to an array or a subarray.  An

array or subarray must be sized with mkAddArrayArea() before

any objects are attached to the array or the array is wired.

Additional mkAddArrayArea() calls will add more area to the array.

The new area is inserted into the array so that its indices

in the array match the lowerLeft through upperRight of the

area.  Any existing area in the array is moved away from the

origin to make room for the new area.

The array is always given a contiguous area between the upperRight

of the area and ( 0,0 ).  Thus area to ((0,0),(4,5)) and

((4,5),(4,5)) have the same effect if the array is unsized.

void

mkPlaceArray( array )

mkId array;

mkPlaceArray() derives the translations of all instances which

are attached or have terminals which are attached to the array,

then interconnects the instances as specified.  Subsequent

operations on the array are legal, but the array must be

placed again before the operations are noticable on the layout.

void

mkFreeArray( array )

mkId array;

mkFreeArray() frees any memory associated with array

and any associated subarrays.  Any further reference to the

array and its subarrays will fail.  See mkOpenSubarray().

void

mkGetGridCoords( array, rowCoords, colCoords )

mkId array;

octCoord *rowCoords;

octCoord *colCoords;

mkGetGridCoords() is used to get the coordinates of grid lines

which separate rows and columns in the array.  This function is

useful for drawing the rows and columns of the array.  The

rowCoords and colCoords arrays must be allocated and passed into

mkGetGridCoords().  They are filled with the OCT coordinates of

the grid lines in the array.

void

mkSetArrayOrigin( array, x, y, relative )

mkId array;

octCoord x;

octCoord y;

int relative;

mkSetArrayOrigin() sets the origin of the array.  The default

origin of the array is at (0,0).  Setting the array origin

effectively moves the array.

Relative states whether the origin is set absolutely to the

x and y coordinates, or relatively to the present position

of the array plus the x and y coordinates.

void

mkSetArrayPitch( array, rowPitch, colPitch )

mkId array;

octBox *area;

octCoord rowPitch;

octCoord colPitch;

mkSetArrayPitch() sets the minimim pitch between row and column

grid lines in the array.  The actual distance between grid lines

in the array is determined by the minimum array pitch and the

instances which are attached to the array in the area bounded

by the lines.

octBox *

mkArrayBB( array )

mkId array;

mkArrayBB() returns the current size of the array, in rows

and columns.

------------------------------------------------------------------

Routines to manage subArrays:

Subarrays are used to identify areas in an array which

should be treated as individual arrays, such as the

AND and OR planes of a PLA.  Once a subarray has been

opened, sized and linked to an array, any references

to the subarray indirectly reference the array.

Subarrays make indexing into areas of the array easier

and make the code more understandable.  An instance can

be attached to OR-PLANE[ 4 ][ 5 ] instead of

PLA[ offset.y + 4 ][ offset.x + 5 ] where offset is the

offset needed to index into the OR-PLANE.

void

mkOpenSubarray( subArray, array )

mkId *subArray;

mkId array;

mkOpenSubarray() opens a subarray of array.

Subarray is set to a unique id which is subsequently

used to reference the subarray.  Once the subarray has

been opened, sized, and linked to the array, references

to the subarray indirectly reference the array.  An array

may have any number of subarrays.

void

mkLinkSubarray( subArray, subArea, area )

mkId subArray;

octBox *subArea;

octBox *area;

mkLinkSubarray() links rows and columns in subArray which

intersect subArea to the rows and columns in subArray's

array which intersect area.  Operations can then be done

on the subArray and are mapped into the proper area in

the array.

Multiple rows or columns in multiple subarrays can be

mapped onto a row or column in the array, but a row or

column in a subarray cannot be mapped onto multiple rows

or columns in the array.

void

mkTranslate( subArray, subArea, area )

mkId array;

octBox *subArea;

octBox *area;

mkTranslate() sets area to the area in the array which is

indirectly referenced by subArea in the subArray.

------------------------------------------------------------------

Routines to attach instances or terms to the array:

void

mkAttach( array, area, object )

mkId arrayId;

octBox *area;

octObject *object;

mkAttach() attaches object to area in array.  The object must be

a valid octObject of type OCT_INSTANCE or OCT_TERM.  An

object may be attached to the array more than once, in which

it will be placed within the bounding box of all the areas

where it is attached to the array.  The object will be checked

for pattern based wiring in the areas it is attached, not in the

area's bounding box.

octBox *

mkObjectBB( array, object )

mkId array;

octObject *object;

mkObjectBB() returns the bounding box of all the areas

that object is attached to array.

------------------------------------------------------------------

Routines to wire the array:

These routines create single segment wiring between terminals

of instances in the array.  Multiple segment wiring, such as

jogs, is presently not available.

There are to mechanisms to specify the wiring, explicit

enumeration of segments with mkNewSegment() or with pattern

based wiring of an area of the array.

When terminals are created, the nets are propogated

as far as possible into the array.  New nets are created

as necessary, but are merged into user specified nets.  Nets

are specified by attaching a formal or actual terminal to the

net before the terminal or its instance is attached to the array.

A symbolic policy error will occur if a terminal is attached to a

net after it has been attached the array.  Mkarray will create a

new net which will not be merged into the user specified net.

The routines to check for this condition would add to the runtime,

so this check was not done.

Several of these routines ( mkNewSegment, mkAddConnectToRule,

and the ruleFacet ) use the notion of layerName, width, and

orientation.  In mkNewSegment and mkAddConnectToRule, these

are parameters to the procedure.  In the ruleFacet, the

layerName is an OCT_LAYER and the width and orientation are

OCT_PROPS.  See mkReadRules for more information on the

ruleFacet.

If layerName is not NIL or "", the segment is attached to the

layer named layerName.  The layer will be created if necessary.

Otherwise the segment will be attached to the highest priority

layer which is common to the two terminal implementations.

If width is MK_MAX_WIDTH (-1), the segment is made the width of

the widest terminal.  If the width is MK_MIN_WIDTH (0), the terminal

is made the minimum feature size of the layer the segment is

attached to, otherwise the segment is made the value of width

wide.  If not MK_MIN_WIDTH or MK_MAX_WIDTH, it is assumed that the

units of width are OCT units, which run 20 to a lambda.

Orientation must be one of 4 values.  If it is MK_VER_SEG or

MK_HOR_SEG, the segment is vertical or horizontal.  If the

orientation is MK_DC_SEG, mkarray will decide whether the

segment is vertical or horizontal.  If it is MK_NONMAN_SEG,

the segment is non-manhattan.

void

mkNewSegment( array, oT1, oT2, layerName, width, orientation )

mkId array;

octObject *oT1;

octObject *oT2;

char *layerName;

int width;

int orientation;

mkNewSegment() will create a segment between oT1 and oT2.  oT1

and oT2 should be either OCT_TERMS or OCT_INSTANCES with a

single actual terminal.

The layerName, width, and orientation are described in the

intro to this section.

void

mkWireArea( array, area, ruleName )

mkWireArea() is used to wire an area of the array by pattern.

The wiring rule name ruleName is enabled and the area of the

array is searched from area->lowerLeft to area->upperRight

for pairs of terminals which are connectible.  Connectible

terminals are in an enabled rule and are either adjacent

or separated by empty space in the array.  Terminals which

are in the same area of the array are connectible if they

are in an enabled rule.

void

mkCreateRule( ruleName )

char *ruleName;

mkCreateRule() creates a rule named ruleName.  Connects must

be added to the rule before it will do anything.

void

mkAddConnectToRule( array, ruleName, oFrom, oTo, layerName, width, orientation )

mkId array;

char *ruleName;

octObject *oFrom;

octObject *oTo;

char *layerName;

int width;

int orientation;

mkAddConnectToRule() adds a connection clause to the rule named

ruleName.  oTo and oFrom must be OCT_TERMS, OCT_INSTANCES with a

single actual terminal, or they may be bags containing OCT_TERMS and

OCT_INSTANCES.  If oTo or oFrom is a bag, everything in the

other object is connected to the contents of the bag.  This is

useful when a set of terminals has a similar set of of connections,

such as when the SOURCE terminals of 5 sizes of transistors must

connect to the DRAIN terminals.   This is done by attaching all

of the SOURCE terminals to oFrom and all of the DRAIN terminals to

oTo ( oFrom and oTo are bags ).

The layerName, width, and orientation are described in the

intro to this section.

void

mkReadRules( array, ruleFacet )

mkReadRules() tells mkarray to read the rules stored in the oct

facet ruleFacet.  The standard cell:view:facet naming convention

applies.  The default view and version is ":symbolic:contents".

The facet must contain representative instances, terminals and

a graph of OCT_BAGS which represent the rules.  The graph is

formed from:

OCT_BAGS named "RULE", which contain string valued OCT_PROPS

named "RULENAME".  These form named rules.

"RULE" bags contain "CONNECT" bags which contain OCT_TERMS,

OCT_INSTANCES with a single term, or OCT_BAGS named

"TERMGROUP" which contain OCT_TERMS or OCT_INSTANCES with

a single term.

"CONNECT" bags may also contain an OCT_LAYER, an integer

valued OCT_PROP named "WIDTH" and an integer valued

prop named "ORIENTATION". These objects have the same

semantics as for mkNewSegment and mkAddConnectToRule().

The facet may also contain a bag named "VERTICAL_LAYERS"

which is ordered and contains the vertical routing OCT_LAYERS

in decreasing priority.

The facet may also contain a bag named "HORIZONTAL_LAYERS"

which is ordered and contains the horizontal routing OCT_LAYERS

in decreasing priority.

The OCT_LAYERS must be attached to the facet and may contain

an OCT_PROP named "WIDTH" which has the same semantics as the

other WIDTH properties and parameters.

------------------------------------------------------------------

Routines to simplify specifying area in the array:

Specifying areas within the array as boxes was choosen

for it's generality. However, boxes can be somewhat clumsy

in practice, so these macros were written.

These macros return boxes from a static array.  Subsequent

calls return the next area in the array.  The size of the

array is AREA_ARRAY_SIZE( see mkarray.h, currently 15 ),

so the results from these macros are good until they are

overwritten after AREA_ARRAY_SIZE subsequent macro calls.

These macros do not return an error code.

octBox *

mkPoint( row, col )

octCoord row;

octCoord col;

octBox *

mkColumn( column )

octCoord column;

octBox *

mkColumns( lower, upper )

octCoord lower;

octCoord upper;

octBox *

mkRow( row )

octCoord row;

octBox *

mkRows( lower, upper )

octCoord lower;

octCoord upper;

octBox *

mkArea( lower, upper, left, right )

int lower;

int upper;

int left;

int right;

octBox *

mkIntersection( area1, area2 )

octBox *area1;

octBox *area2;

octBox *

mkUnionBB( area1, area2 )

octBox *area1;

octBox *area2;

mkPoint() converts row and column into an area.

mkColumn() converts column into an area.

mkColumns() converts a range of columns into an area.

mkRow() converts row into an area

mkRows() converts a range of rows into an area

mkArea() packages lower, upper, left, and right into an area.

mkIntersection() returns the intersection of two areas.

mkUnionBB() returns the bounding box of the union of two areas.

------------------------------------------------------------------

void

mkWireIntraInstance( array, value )

mkId array;

int value;

mkWireIntraInstance( array, 1 ) sets a mode where terminals

within an instance will match during pattern based wiring.

The instance is then wired to itself.  Be careful, this may

cause overconstraints and/or layout error.

mkWireIntraInstance( array, 0 ) turns off this wiring mode.

void

mkSetDebug( array, value )

mkId array;

int value;

mkSetDebug( array, 1 ) sets a debugging mode with more error

checking and an overconstraint check before the constraint

graph is solved.

mkSetDebug( array, 0 ) turns off the debug mode.

void

mkSetExpansion( array, value )

mkId array;

double value;

mkSetExpansion( array, N ) tells mkarray to expand the bounding

box of each instance by N.  This effectively spaces the instances

so that the size of the array is increased by a factor of N.

If instances in the array cover more than one row or column, this

routine may cause the constraint graph to be overconstrained.

Be careful.  The default expansion is 1.0.

void

mkSetPrintGraph( array, value )

mkId array;

int value;

mkSetPrintGraph( array, 1 ) causes the constraint graphs to be

printed in a form readable by the CPM constraint graph

solver.  The resulting files are named mkarray.x and mkarray.y.

------------------------------------------------------------------

Error mechanisms:

Unless stated otherwise the mkarray procedures do not return

anything.  If an error occurs, it is trapped with