A library consists of a number of gates.  All of the gates which have

the same logic functions are grouped into 'classes'.  The basic data

types are library_t, lib_class_t, and lib_gate_t.  It is verboten to

look inside these structures, because the organization of the data is

still being refined.

library_t *

lib_get_library()

Returns the 'current' library, or NIL(lib_library_t) if a current

library has not been defined.

lsGen

lib_gen_classes(library)

library_t *library;

Initialize an lsGen generator for the class in a given library.

See generator usage example below.

lsGen

lib_gen_gates(class)

lib_class_t *class;

Initialize an lsGen generator for the gates in a given class.

See generator usage example below.

lib_class_t *

lib_get_class(network, library)

network_t *network;

library_t *library;

Given a 'network', find the gate-class which has the same logic

function.  Returns NIL(lib_class_t) if there is no such gate in

the library.  A fast way to construct a network is using

read_eqn_string().

lib_gate_t *

lib_get_gate(library, name)

library_t *library;

char *name;

Get a gate by its name.  Returns NIL(lib_gate_t) if no gate with

that name is found in 'library'.

/*

*  Attributes of a class

*/

char *

lib_class_name(class)

lib_class_t *class;

Returns the name of a class.  Currently, a class is named by the

name of its first gate (essentially, a randomly chosen gate).

lib_class_t *

lib_class_dual(class)

lib_class_t *class;

Returns the dual class of a class, or NIL(lib_class_t) if the

given class does not have any dual gates in the library.

network_t *

lib_class_network(class)

lib_class_t *class;

Return a 'network' which represents the logic function associated

with a class.  Do not alter or free this network.

/*

*  Attributes of a gate

*/

char *

lib_gate_name(gate)

lib_gate_t *gate;

Returns the name of a gate.  Do not free or alter the return string.

char *

lib_gate_pin_name(gate, pin, inflag)

lib_gate_t *gate;

int pin;

int inflag;

Returns the name of a pin on a gate.  Do not free or alter the

return string.  Returns NIL(char) if 'pin' is more than the

number of pins on the gate.  If 'inflag' is 1, then the n'th

input pin name is returned; if 0, then the n'th output pin name

is returned.  Pin numbers start at 0.

double

lib_gate_area(gate)

lib_gate_t *gate;

Return the area of a gate.

lib_class_t *

lib_gate_class(gate)

lib_gate_t *gate;

Returns the class of a gate.

int

lib_gate_num_in(gate)

lib_gate_t *gate;

Returns the number of inputs of the gate.

int

lib_gate_num_out(gate)

lib_gate_t *gate;

Returns the number of outputs of the gate.

/*

*  Mixing the library and node packages

*/

lib_gate_t *

lib_gate_of(node)

node_t *node;

Return the gate which is currently implementing 'node'.  Returns

NIL(lib_gate_t) if this node is not mapped.  Note that PRIMARY_INPUT

and PRIMARY_OUTPUT nodes are never mapped; hence, lib_gate_of()

will always return NIL(char) for them.

int

lib_network_is_mapped(network)

network_t *network;

Returns 1 if the network is mapped (i.e., all nodes in the network

have an associated gate from a library), or 0 if not.

int

lib_set_gate(node, gate, formals, actuals, nin)

node_t *node;

lib_gate_t *gate;

char **formals;

node_t **actuals;

int nin;

Sets the implementation of the given node to be 'gate'.  This

changes the logic function of the node, and sets the node as

'mapped'.  'formals' is an array of names of the formal

terminals of the gate (i.e., the gate pin names).  'actuals' is

an array of nodes in the network (should be the same network

that 'node' belongs to) which is matched in 1-1 correspondance

with 'formals'.  As a consistency check, 'nin' declares the

sizes of the arrays 'formals', and 'actuals'.  Returns 1 on

success, 0 if any error.  Probable errors are:  gate is

NIL(lib_gate_t), number of inputs declared via 'nin' does not

match the number of inputs on 'gate', the specified gate is not

single-output, or some name in 'formals' is not a pin on the

gate.  error_string() is appended to in the case of an error.

Generator usage example:

lsGen gen, gen1;

lib_library_t *library;

lib_class_t *class;

lib_gate_t *gate; char *dummy;

library = lib_get_library();

gen1 = lib_gen_classes(library);

while (lsNext(gen1, &dummy, LS_NH) == LS_OK) {

class = (lib_class_t *) dummy;

/*

*  do something with class

*/

gen2 = lib_gen_gates(class);

while (lsNext(gen2, &dummy, LS_NH) == LS_OK) {

gate = (lib_class_t *) dummy;

/*

*  do something with gate

*/

}

(void) lsFinish(gen2);

}

(void) lsFinish(gen1);