Bus Policy Alternatives

David Harrison

Bus: A possibly heirarchical collection of wires.  Each bus may

contain a number of other busses.  Leaf busses contain a

collection of single connections.

Purpose:

1. Data used by netlisting tools to derive the connectivity of

schematic designs that use busses.

2. Annotation in schematic diagrams used for design

documentation.

Requirements:

1. Extension of symbolic connectivity policy.

2. Minimize work required of external netlisters for extracting

useful connectivity from schematic diagrams.

3. As simple to maintain and store as possible.

Base Policy:

Like symbolic policy,  connectivity is represented using nets

attached to segments that implement the nets.  All segments are

two-point paths.  Segments are attached to the actual terminals

of their end-point connectors.  Actual terminals of non-connector

instances are attached to a net(s).  Actual terminals of connector

instances are not attached to a net(s).

Rippers:

A special form of connector instance that can accept a bus at one

terminal and produce a portion of the bus on another terminal.

The portion of the bus ripped off is specified by annotation on

the interface facet of the ripper cell master.

Alternatives:

1. A number of nets all attached to the same implementation.  For

example,  an eight-bit bus would be represented as eight nets

all sharing the same implementation.  All nets would be

connected to one actual terminal on each instance/ripper.

2. Multiple actual terminals of instances.  Like above, a number

of nets all attached to the same implementation would be used

to represent a bus.  However, each net would be connected to a

unique terminal on an instance.  This implies instances would

have several actual terminals that are used to represent one

logical bus connection.

3. Annotation of a single net.  An eight-bit bus might be

represented as a single net with annotation indicating its

width is eight bits.