/* newG.doc --

*

*     George Carvalho, Esq.

*     (Advising Professor - E. S. Kuh)

*

* The newG (new global router) module tries to construct a

* steiner tree for each net and then goes through this tree

* and makes a list of all the wall junctions in it.  This

* information is used by the newP (new placement) module.

*/

SampleUsage()

{

CellMaster          *cellMasterPtr;

Ngr_BldTreeDataPtr  bldTreePtr;

Gt_Graph            baseGraph, compGraph;

Mac_GraphData       *graphDataPtr;

Net                 *netPtr;

Ngr_BldTreeBegin(cellMasterPtr, NIL(Ngr_BldOptions), &bldTreePtr);

foreach net {

Ngr_BuildBaseGraph(bldTreePtr, netPtr, &baseGraph);

/*

* The next two steps will be automatically skipped if

* the number of pins in the net is greater then

* NGR_UPPER_PIN_CUT_OFF.  If this happens baseGraph

* will contain an ordinary tree (probably with a lot of

* extra edges) instead of a steiner tree after the last step.

*/

Ngr_BuildCompGraph(bldTreePtr, &compGraph);

Ngr_FindMST(bldTreePtr);

Ngr_FindSteinerTree(bldTreePtr);

graphDataPtr = (Mac_GraphData *) Gt_GetGraphDataOfGraph(baseGraph);

/*

* At this point baseGraph contains a steiner tree (whose edges are

* not necessarily all manhattan), and 'graphDataPtr->junctList'

* contains a list of the wall junctions which correspond to the

* switch box areas.  Note however that 'graphDataPtr->bnTileList'

* is not constructed and so is always NULL.

*/

Ngr_DeleteBaseGraph(&baseGraph);

Ngr_DeleteCompGraph(&compGraph);

}

Ngr_BldTreeEnd(bldTreePtr);

}