Region Library

David Harrison

University of California,  Berkeley

Below are the specifications of the utililty routines available

in the region package.  You must include `region.h' and `oct.h'

before using the functions described below:

Generating Objects in a region:

------------------------------

regStatus regObjStart(theFacet, region, mask, theGen, allInside)

octObject *theFacet;		/* Facet to look in            */

struct octBox *region;		/* Region to search            */

octObjectMask mask;		/* Geometry mask               */

regObjGen *theGen;		/* Generator handle            */

int allInside;			/* If set, only those enclosed */

/*

* This routine initializes the generation of geometry in

* the given region.  The difference between this generator

* and the region package is this generator automatically

* generates across all layers in the view to find the geometries.

* Only items which have bounding boxes will be found using

* this routine.  If the flag 'allInside' is set,  only geometries

* entirely inside the region will be returned (normally all

* objects intersecting the region are returned.  The routine

* may return REG_NOMORE if there aren't any objects to generate.

*/

regStatus regObjNext(theGen, theObj)

regObjGen theGen;		/* Generator handle */

octObject *theObj;		/* Returned object  */

/*

* This routine generates the next object in the sequence

* initialized by utInitObjects.  If there are no more objects,

* the routine returns REG_NOMORE and leaves 'theObj' unchanged.

*/

void regObjFinish(theGen)

regObjGen theGen;

/*

* This routine ends the generation of geometry initialized by

* utInitObjects.  This routine should be called after the

* caller has gotten all information desired from the generation

* sequence.  NOTE:  this routine is NOT called automatically

* once all items have been generated.

*/

Symbolic Policy Assistance

--------------------------

regStatus regFindActual(theFacet, thePoint, termObject, implBB)

octObject *theFacet;		/* Facet of the point          */

struct octPoint *thePoint;	/* Point in OCT coordinates    */

octObject *termObject;		/* Returned terminal           */

struct octBox *implBB;		/* Implementation bounding box */

/*

* This routine finds the actual terminal at 'thePoint'.  If there

* is no such terminal or the terminal is ambiguous the routine

* returns REG_FAIL.  The routine also returns a bounding

* box which contains the implementation of the terminal.  Note it

* assumes the terminals are implemented using boxes. Here are the steps:

*   I.  For each instance under 'thePoint':

*       A. Open up the interface of the instance.  This MUST exist

*          (an error results if it doesn't).

*       B. Translate 'thePoint' into the interface coordinate system.

*       C. For each object under the translated point:

*          1. If the object implements a formal terminal,  save it's name.

*          2. If another name has already been saved,  the terminal

*             is not unique.  This is an error.

*       D. Save the terminal name at the instance level.  If another

*          name is already saved,  the terminal is ambigious.  The

*          routine barfs.

*   II. Given the unique name of the terminal,  generate the

*       actual terminal and return.

*/

regStatus regMultiActual(theFacet, extent, n_ok, ary)

octObject *theFacet;		/* Facet of the point        */

struct octBox *extent;		/* Area to look for terminal */

int *n_ok;			/* Number of terminals found */

regTermInfo **ary;		/* Returned info             */

/*

* This routine finds all actual terminals which intersect

* the box `extent' in facet `theFacet'.  It returns

* all of the terminals and their associated bounding boxes

* in the array `ary'.  The number of terminals returned is

* given by `n_ok'.  The space for the array is allocated

* in this routine and should be freed once the caller is

* finished with it.  It will return REG_FAIL if no terminals

* are found.

*/

regStatus regFindImpl(theTerm, implBB)

octObject *theTerm;		/* Input terminal          */

struct octBox *implBB;		/* Implementation geometry */

/*

* This routine finds the bounding box of the implementation

* of some terminal.  There are two cases:  formal and actual

* terminals.  For formal terminals,  we look at its implementation

* directly and return that.  If it is an actual terminal,  then

* we hand that to the actual terminal finder which  opens up

* master's interface and finds the geometry there.

*/

regStatus regFindLyrImpl(theTerm, implBB, count, lyrAry)

octObject *theTerm;		/* Input terminal          */

struct octBox *implBB;		/* Implementation geometry */

int *count;			/* Return count of lyrs    */

char ***lyrAry;			/* Returned array          */

/*

* This routine finds the bounding box of the implementation

* of some terminal.  There are two cases:  formal and actual

* terminals.  For formal terminals,  we look at its implementation

* directly and return that.  If it is an actual terminal,  then

* we hand that to the actual terminal finder which  opens up

* master's interface and finds the geometry there.  The difference

* between this routine and regFindImpl is this one returns all

* of the layers of the implementation as well.

*/

regStatus regFindNet(obj, netObj)

octObject *obj;			/* Object implementing net */

octObject *netObj;		/* Returned net            */

/*

* This routine searches for the major net associated with

* 'obj'.  The object can be a path or an actual terminal.

* If it finds a unique net,  it copies it into 'netObj'.

* The routine will return REG_FAIL if the net can't be

* found,  is not unique,  or the object cannot implement

* a net.

*/

typedef struct reg_term_info {

octObject term;		/* Terminal     */

struct octBox bb;		/* Bounding box */

} regTermInfo;

regStatus regSegActuals(theFacet, lyr, p1, p2, n_ok, ary, n_fail)

octObject *theFacet;		/* Facet of the path   */

octObject *lyr;			/* Conditional layer   */

struct octPoint *p1;		/* First point         */

struct octPoint *p2;		/* Second point        */

int *n_ok;			/* # rtned terminals   */

regTermInfo **ary;		/* Returned info       */

int *n_fail;			/* Total terms found   */

/*

* This routine finds the actual terminals which intersect a path drawn from

* `p1' to `p2' (it assumes no width) on layer `lyr'.  It returns all of the

* terminals and their associated bounding boxes in the array `ary'.  The

* number of these terminals is returned in `n_ok'.  These are

* dynamically allocated arrays which should be freed after the caller is

* finished with them.  The mechanism for finding such terminals is VERY

* involved and is summarized in the routine regFindActual.  This routine is

* made more complex by the fact it must find all terminals intersecting the

* path.  The total number of terminals found (including those rejected

* due to the layer test) is given in `n_fail'.

*/

Low-Level Interface

-------------------

Below is the low level interface to the region package.  The

package will use k-d trees to increase lookup performance if

there are enough objects to warrant it.

void regInit(parentObj, someExtent, someMask, generator)

octObject *parentObj;		/* OCT object to generate objects from */

struct octBox *someExtent;	/* Rectangular region in OCT coords    */

octObjectMask someMask;		/* OCT Mask of objects to look for     */

regGenerator *generator;	/* Returned generator                  */

/*

* This routine returns a generator handle which can be used to

* retrieve the objects of type `someMask' that intersect `someExtent'.

* The routine supports all geometric objects (OCT_INSTANCE, OCT_POLYGON,

* OCT_BOX, OCT_CIRCLE, OCT_PATH, OCT_LABEL, OCT_POINT, and OCT_EDGE),

* and one non-geometric object: OCT_TERM.  The size of a formal terminal

* is defined as the union of the sizes of all geometry attached to

* it.  As a special case,  if a terminal has an actual terminal attached

* to it,  its size will be determined by the size of all items attached

* to the interface formal terminal corresponding to the actual terminal

* in the instance master.  The region specified should be relative

* to the facet containing `parentObj'.

*/

regStatus regNext(generator, object, num_points, points, size)

regGenerator *generator;	/* Generation Identifier      */

octObject *object;		/* Returned object            */

int *num_points;		/* Number of points returned  */

struct octPoint **points;	/* Points for path or polygon */

struct octBox *size;		/* Returned size of object    */

/*

* This routine returns the next OCT object in a generation sequence

* identified by 'generator'.  If the object is a path or a polygon,

* and `num_points' and `points' are non-zero,  `num_points' will be

* filled with the number of points and `points' will be filled with

* the object points.  The buffer returned in `points' is owned by the

* region package and should not be modified.  This buffer may be

* overwritten the next time regNext() is called.  If `size' is

* non-zero, the bounding box of item will be returned.  If there

* are more objects to be generated in the region,  the routine

* will return REG_OK.  If there are no more items to generate,

* the routine will return REG_NOMORE.  regEnd() should always be

* used to terminate the generation sequence.

*/

void regEnd(generator)

regGenerator *generator;	/* Generation Identifier */

/*

* This routine frees the resources associated with 'generator'.  It

* can be called anytime after the initialization of the identifier.

* Since regNext does not automatically free the resources,  this

* routine should always be called after a generation sequence.

*/

regStatus regGetPoints(obj, n, points)

struct octObject *obj;		/* Path or polygon object    */

int *n;				/* Number of points          */

struct octPoint **points;	/* Points of polygon or path */

/*

* This routine gets the points associated with a path or polygon

* object.  The returned data is owned by this routine and should

* not be freed.  The routine guarantees that the returned area

* is at least n+1 spaces long in case the polygon must be closed.

* It is a precondition that the object is a path

* or polgon and the routine does not check to make sure this is true.

* The routine returns a non-zero value if there were no errors.

*/

Error Handling

--------------

Routines returning a regStatus return one of the following status

codes (defined in region.h):

#define REG_OK		1	/* Successful completion */

#define REG_FAIL	0	/* Error occurred        */

#define REG_NOMORE	-1	/* No more items         */

If a region routine returns REG_FAIL,  the following function can be

used to obtain the reason for failure:

char *regErrorString()

/*

* Returns an textual error message describing the reason for

* the failure of some region routine.

*/

In the unlikely event of a fatal error,  the region package calls an

error handler provided by the errtrap package (see errtrap.h).

Unless overridden,  the standard error handler prints a message and

exits.  Below is a list of fatal errors generated by the region

package (the codes are available in region.h):

REG_MEM ("out of memory")

No more memory is available.

REG_OCT ("oct error: %s:\n  %s")

Some unexpected oct error has occurred.  The first %s will be

replaced by a region specific message.  The second %s will be

replaced by a Oct specific message.

REG_UCR ("bad change record")

While maintaining the k-d representation of geometry,  the

region package encountered a serious change list

inconsistency.

REG_BA ("bad argument: %s")

A bad argument was passed to a function.  The offending

function will be included in %s.

REG_NIF ("cannot open interface facet:\n  %s")

The region package could not examine the interface facet of

the master of some instance.  An oct specific error message

will be given in %s.

REG_CON ("inconsistent facets: %s\n  %s")

The region package detected an inconsistency between the

contents and interface facet of a cell.  Usually,  this means

the contents facet has been changed without properly updating

the interface facet.

REG_UNKNOWN ("unknown error %d")

Some unknown error has occurred.