Channels:

Channels are grouped in a binary tree as follows:

Each channel represents a submodule and a generalized partition of itself.

The partition represents the "channel" subdividing the submodule.

channel name = "CDChanDef" (type==OCT_BAG)

contains:

if a real channel:

channel order property

sub channels

sub nets

instance corner edges

if a leaf channel (no partition):

cell instance

surrounding global wire graph description

containers:

channel (if not root channel)

contents facet (if root channel)

Channels have a order property.  This property designates the order in which

the channels are to be routed.

channel order name = "CDChanOrder"   (type==OCT_BAG, prop.type==OCT_INTEGER)

prop.value = order which the channels are to be routed (0 = 1st channel

to be routed, n = (n+1)th channel to be routed).

e.g.

The macro cell below is represented by the tree on the right:

+----------+.....+--------+                 ************

|          |     |        |                 * contents *

|          |     | Inst 2 |                 *   facet  *

|          |     |        |                 ************

|          |  C  +--------+                       |

|          |  h  .        .                  .....V.....

|  Inst 1  |  a  . Chan B .                  . Chan  A .

|          |  n  .        .                  ...........

|          |     +--------+                   |       |

|          |  A  |        |         ..........V..   ..V........

|          |     | Inst 3 |         . Leaf Chan .   . Chan  B .

|          |     |        |         .............   ...........

+----------+.....+--------+            |              |     |

+-----V--+ ..........V.. ..V..........

| Inst 1 | . Leaf Chan . . Leaf Chan .

+--------+ ............. .............

|             |

+---V----+  +-----V--+

| Inst 3 |  | Inst 2 |

+--------+  +--------+

Global route subnets:

Nets are grouped into channels as follows:

1) A subnet corresponding to channel is attached under the net

2) All aTerms in the net/channel are attached under the subnet.

The original net - aterm attachments are untouched.

3) Pseudo terminals (terminals of the subnet representing net channel to

channel interfaces) are created and attached to both the original

net and the subnet.

4) Subnets are then attached under its corresponding channel.

channel subnet name  = "CDSubNet"     (type==OCT_NET)

subnet contains:

actual terminals    (real terminals)

psuedo terminals    (channel to channel terminals)

subnet containers:

original net

channel subnet traverses

psuedo terminal name = "CDPseudoTerm" (type==OCT_BAG)

psuedo terminal contains:

net width property

psuedo terminal containers:

subnet

original net

The net pin width is attach to pseudo terminals and actual terminals of special

net pins that are not to be routed by a standard minimum width wire.

net terminal width name = "PorGWidth" (type==OCT_PROPERTY,

prop.type==OCT_INTEGER)

e.g.

A global wire net below is represented by the tree on the right:

..........

. Chan A .----+

..........    |

|

..........    |

.      .                                            . Chan B .    |

.      .                                            ..........    |

.Chan B.                         *************             |      |

.      .                         *  Net      *----------+  |      |

ATerm 3--+   .                         *************-------+  |  |      |

.  |   .                          |  |  |  |         |  |  |      |

.  |PTerm      ATerm     *********V* |  |  |         | .V..V..... |

......4.............2..     * ATerm 3 <-|--|--|---------|-. SubNet . |

|             |       *********** |  |  |  +------|-.......... |

Chan A   +-+-----------+                   |  |  |  |      |            |

|                      .........V. |  |  |      |            |

........1..............        . PTerm 4 <-|--|--+      |            |

ATerm                    ..........<-|--|----+    |            |

|  |    |    |            |

*********V* |    +---.V........    |

* ATerm 1 <-|--------. SubNet <----+

*********** |    +---..........

|    |

*********V*   |

* ATerm 2 <---+

***********

Global wire graph:

Surrounding global wire graph description

graph description name = "CDGraph" (type==OCT_BAG)

contains:

1st point of surrounding polygon

containers:

leaf channel

Surrounding polygon is an ordered list of verticies that are connected together

with bags representing the graph edges.

The verticies are oct points and the edges are oct bags.

The graph edges represent global wiring channels.

The graph edges have and edge width property representing the width of the

global wiring channel.  The graph edges are attached to its corresponding

routing channel.

graph edge name = "CDGraphEdge" (type==OCT_BAG)

contains:

points  (representing end points - last/closing endpoint is assumed to

be the first point attached to the graph description bag and is not

attached to that point).

global wire channel width property

containers:

channel (routing channel corresponding to global wiring channel - a

many to 1 surjective mapping)

global wire channel width name = "CDEdgeWidth" (type==OCT_PROPERTY,

prop.type==OCT_INTEGER)

prop.vale = average width of global wiring channel in database units

Corners:

Corners of instance boundaries designate each instance boundary segment to

a routing channel.  The boundaries of an instance are assumed to be either

clockwise or counterclockwise (i.e. does not cross itself).

A boundary path delineating part of the routing channel defined by a bordering

instance is defined by two corners and an corner edge attached to the

routing channel.

Each corner is an index into an array of points (representing the boundary of

an instance).

corner bag name = "CDCorners" (type==OCT_BAG)

contains:

1st corner of instance boundary

containers:

instance

corner name = "CDCorner" (type==OCT_PROPERTY, prop.type==OCT_INTEGER)

prop.value = index into oct pt array of instance master "place" layer

polygon.  However if instance master "place" layer bounding

polygon is a box then:

0 represents the lower left corner of the master box,

1 represents the lower right corner of the master box,

2 represents the upper right corner of the master box,

3 represents the upper left corner of the master box.

corner edge = "CDCornerEdge" (type==OCT_BAG)

contains:

corner property

containers:

channel