Berkeley Logic Interchange Format (blif)

University of California

Berkeley

May 28, 1987

The goal  of blif  is to  describe a  Boolean network  in

textual form.     A Boolean  network  is  a  representation

of an  arbitrary combinational  logic  network, and  is  an

acyclic directed graph  with a logic  function attached  to

each node.  Each  node in this representation has a  single

output.  Therefore, each net (or signal) has only a  single

driver, and we can therefore name either the signal or  the

gate which drives the signal without ambiguity.

In this  section, angle-brackets  surround  nonterminals,

and vertical bar separates  choices in a pseudo-BNF  style.

Bdsyn uses only  the logic-gate construct of  blif.   Bdsyn

uses the  the keywords  .module, .inputs,  .outputs,  .end,

and .names.  The module-reference construct of blif is  not

implemented.

1 Modules

A module is declared by:

.module <decl-module-name>

.inputs <decl-input-list>

.outputs <decl-output-list>

<logic-gate> | <pla-reference> | <module-reference>

.

.

.

<logic-gate> | <pla-reference> | <module-reference>

.end

decl-module-name  is a  string  giving the  name  of  the

module.

decl-input-list  is  a   white-space-separated  list   of

strings (terminated  by the  end of  the line)  giving  the

formal parameters for the module  being declared.  If  this

is the  last or  only  module, then  these signals  can  be

identified as the primary inputs of the circuit.   Multiple

.inputs lines  are allowed,  and the  lists of  inputs  are

merely concatenated.

decl-output-list  is  a  white-space-separated  list   of

strings (terminated  by the  end of  the line)  giving  the

1

formal parameters  for  the  module being  declared.     If

this  is the  last  or  only  module,  then  these  signals

can be identified  as the primary  outputs of the  circuit.

Multiple .outputs  lines  are  allowed, and  the  lists  of

inputs are merely concatenated.

It is expected that a  blif parser may allow the  .inputs

and .outputs  statements  to be  optional.    If  they  are

not specified,  the  primary inputs  can be  inferred  from

the  signals  which  are  not  the  outputs  of  any  other

logic  block,  and the  primary  outputs  can  be  inferred

from the  signals which  are not  the inputs  to any  other

blocks.     This  makes  the  serious  assumption  that  no

primary output is  used as an intermediate  signal.   Also,

these assumptions are  only useful for  the "root"  module.

Subcircuits  are  invoked  with  positional  arguments  and

hence the specified order for the inputs is significant  in

a subcircuit.

The keyword .end is optional,  and is implied at the  end

of the file.

Anywhere in  the  file, a  `\'  (backslash) as  the  last

character  on  a  line   indicates  concatenation  of   the

subsequent line to the current line.

2 Logic Gates

A logic-gate is:

.names <in-1> <in-2> ... <in-n> <output>

<single-output-cover>

single-output-cover   is   an  n-input,    1-output   PLA

description of  the  logic function  corresponding  to  the

logic gate.   f0, 1, --g is used  in the "input plane"  and

f0, 1, --,  ~g is used in  the "output plane".   The  logic

gate can have  only its  ON-set specified with  1's in  the

"output plane", or  can also have a  DC-set and an  OFF-set

specified with -'s or 0's in  the "output plane".  ~  means

no connection.  For a more complete description of the  PLA

input format, see espresso(5).

3 Module (subcircuit) references

A module-reference is

.subckt <module-name> <input-list> <output-list>

module-name   gives  the   name   of   a   module   being

included.      It  must  be   previously  defined  in   the

file.    input-list  are  the  actual  parameters  and  are

matched one-to-one with  the corresponding  decl-input-list

2

inside  the module  declaration.     Likewise,  output-list

are  the  actual  parameters  and  are  matched  one-to-one

with the corresponding  decl-output-list inside the  module

declaration.

input-list  and  output-list  are  white-space  separated

sequence of  tokens ending with  the end  of the line.    A

"/" may be  used to separate  input-list from  output-list.

The  end  of  input-list  and  the  start  of   output-list

is, however,  always  inferred from  the number  of  inputs

and  outputs  in   the  module  declaration   (i.e.,   from

decl-input-list and  decl-output-list of  the module  which

is being called).

thefformaliparameters withintthe-modulerdeclarationn,become

the actual parameters.   It is  an error if the  input-list

and output-list  are  given and  the  number of  inputs  or

outputs does not match the "call" and the definition.

4 PLA References

A pla-reference is

.pla <pla-name> <input-list> <output-list>

pla-name  gives the  name  of  a file  containing  a  pla

description  in  the  format  understood  by   espresso(5).

input-list and  output-list are matched  1 for  1 with  the

inputs and outputs  of the PLA  (which, for arcane  reasons

are given  via  .ilb  and .ob  keywords  within  the  PLA).

Similar to  .subckt,  a  "/" may  separate  the  input-list

and the  output-list and if  both are  missing, the  formal

parameters become the actual parameters.

3