Geometric Updates

David Harrison

University of California, Berkeley

1989

This package provides an interface above Oct change lists for

examining geometric changes.  Geometric changes are defined

as changes to objects of type OCT_GEO_MASK and OCT_INSTANCE_MASK.

Changes to these objects are automatically collapsed down

to a minimum for quick analysis.

Setting Markers

---------------

gu_marker guMark(container, mask)

octObject *container;		/* Object container    */

octObjectMask mask;		/* Interesting objects */

This routine creates a new marker for examining changes to geometric

objects attached to `container'.  Changes will be monitored from the

time the marker is created.  Only changes to geometric objects

(OCT_GEO_MASK|OCT_INSTANCE_MASK) whose type is in `mask' will

be monitored.  A marker created using this routine should

be removed using guUnmark().

void guUnmark(v_mark)

gu_marker v_mark;	/* Free marker */

Frees resources associated with `mark'.  The freed marker

should not be referenced again.

Generating Changes

------------------

gu_gen guStart(v_mark)

gu_marker v_mark;	/* Marker */

Begins a generation of changes to geometric objects meeting the

criteria specified to guMark() at mark creation time.  The

routine returns a handle that should be used in calls to

guNext() to generate the changes.  Only changes up to the time

guStart() is called will be generated by guNext().  After generating

through the changes,  the caller should call guEnd() to reclaim

the necessary resources.

gu_change guNext(v_gen)

gu_gen v_gen;		/* Generator state */

This routine generates the next change in the sequence of

changes initialized by guStart().  An opaque change record

is returned by the function.  It can be examined by

using the functions guOp(), guXid(), guOldSize(),

guOldObject(), and guOldPoints().  When there are no

more changes,  the routine returns zero. The generator state

is not automatically reclaimed.  Thus,  the user should

call guEnd() after generating through the changes.

void guEnd(v_gen)

gu_gen v_gen;

Frees up all resources associated with the change generator

`gen'.  It is important that the user use this call to reclaim

the resources consumed by the generation sequence.  These

resources are substantial in this implementation.

Examining a Change

------------------

gu_op guOp(v_change)

gu_change v_change;

Returns the type of `change'.  The type is one of the

following:

GU_ADD:    An item was added to the container specified in the

call to guMark().  This may have been either an

octAttach or an octCreate().

GU_UPDATE: An item attached to the container specified in

the call to guMark() was modified.  This may have

been any number of octModify or octPutPoint

operations.

GU_REMOVE: An item attached to the container specified in

the call to guMark() was removed from that

container.  This may have been either an

octDetach or an octDelete operation.

int32 guXid(v_change)

gu_change v_change;

Returns the external id of the changed item.  Together with

guFacet(), this can be used to obtain the changed item using

octGetByExternalId() if the object still exists.

octObject *guFacet(v_change)

gu_change v_change;

Returns the facet of the change record.  If the facet has

ceased to exist for some reason,  the routine returns zero.

The object returned is static and should be copied if the

caller wishes to preserve its contents across multiple

calls to guFacet().  This is meant to be used with guXid() for

obtaining a copy of the changed object if it still exists.

struct octBox *guOldSize(v_change)

gu_change v_change;

Returns the old size of the item *before* any change occured

to it.  This is useful for determining the area affected by

the change.  Under some conditions, the old size may not be

computed and the routine will return zero.  See the

programming example for sample use of this information.

octObject *guOldObject(v_change)

gu_change v_change;

Returns the old object associated with a record (if any).

Initially unimplemented.

int guOldPoints(v_change, pnts)

gu_change v_change;

struct octPoint **pnts;

Returns the old points associated with an object (if any).

Initially unimplemented.

Error Handling

--------------

When a serious error occurs (e.g., running out of memory) the gu

package calls the fault handling routine provided by the `errtrap'

error handling package (see errtrap.doc for details).  Unless

overridden, the default error handler will print an error message and

abort the program.  A description of serious gu errors is given below:

GU_NO_BAG ("Cannot create bag for change list")

The change lists for the package are stored in a temporary bag

attached to the facet.  This error indicates the package could

not create that bag.

GU_NO_LIST ("Cannot create change list")

The package could not create an OCT_CHANGE_LIST for tracing

changes to the database.

GU_NO_DEL ("Cannot delete change list")

The package could not delete a change list used for tracing

changes to the database.

GU_BAD_CL ("Cannot generate records off change list")

The package could not generate database changes for analysis.

GU_BAD_OP ("Invalid op")

Indicates the package encountered a database change record it

was not expecting.

GU_BAD_ID ("Can't get change record by id")

Indicates the database failed to retreive a change list for

analysis.

GU_BAD_REC ("Bad change list record")

The database returned a change record of an unknown type.

GU_NOT_IMPL ("Not implemented")

The requested functionality is not implemented yet.

Programming Example

-------------------

/*

* The small code fragment below shows how this package can be used to

* trace geometric changes.  The function `incr_drc' shows how one might

* implement an incremental design rule checking system using the gu

* package.  This is not meant to be a full example.  An actual system

* would be more complex.

*/

#include "gu.h"

#include "oct.h"

void incr_drc(facet)

octObject *facet;

/* Check the changed areas of `facet' for design rule violations */

{

foreach(facet, OCT_LAYER, examine_layer);

}

void examine_layer(layer)

octObject *layer;

{

gu_marker marker;

gu_gen gen;

gu_change change;

octObject object;

struct octBox new_location;

if (marker = fetch_marker(layer)) {

gen = guStart(marker);

while (change = guNext(gen)) {

/* Check place where it was */

check_area(layer, guOldSize(change));

/* Check place where it is (if any) */

if ((octGetByExternalId(guFacet(change),

guXid(change), &object) == OCT_OK) &&

(octBB(&object, &new_location) == OCT_OK)) {

check_area(layer, &new_location);

}

}

guEnd(gen);

guUnmark(marker);

} else {

/* Check the whole layer */

check_all(layer);

}

/* Generate new marker to see changes from now on */

update_marker(layer, guMark(layer, OCT_GEO_MASK));

}

void foreach(container, mask, function)

octObject *container;

octObjectMask mask;

void (*func)();

/* Generates contents of `container' of type `mask' and calls `function' */

{

/* Left to the reader ... */

}

gu_marker fetch_marker(layer)

octObject *layer;

/* Looks up the marker stored for `layer' in some sort of table */

{

/* Left to the reader ... */

}

void update_marker(layer, marker)

octObject *layer;

gu_marker mark;

/* Updates the marker stored for `layer' to `marker' */

{

/* Left to the reader ... */

}

void check_area(layer, area)

octObject *layer;

struct octBox *area;

/* Looks for design rule errors in `area' of `layer' */

{

/* Left to the reader */

}

void check_all(layer)

octObject *layer;

/* Looks for design rule errors on `layer' */

{

/* Left to the reader */

}