BDSYN Users' Manual

Version 1.1

Russell B. Segal

University of California

Berkeley

May 22, 1987

1 Introduction

Bdsyn is a  hardware description translator.   It takes  as

input a  textual description  of a  block of  combinational

logic and produces  a collection of  logic functions  which

implement  the described  function.     Bdsyn  was  written

to  be a  front  end  to  the  Berkeley  Synthesis  System.

Work  on bdsyn  was  begun  in  the spring  of  1986  as  a

class project  for Berkeley's  synthesis class.    For  the

synthesis class, we  were given  permission to use  Digital

Equipment Corporation's  proprietary multi-level  simulator

decsim.   The functional  simulation language  bds is  used

for high level  simulation in decsim.   We  have chosen  to

use a  subset of  the bds  language as  the input  language

to bdsyn.   The output of  bdsyn is a multiple-level  logic

representation of  the  described  function.    The  output

created is  in blif  (Berkeley Logic  Interchange  Format).

This  output  is  suitable  input  for  mis  which  is  the

multiple level  logic minimizer in  the Berkeley  Synthesis

System.

1.1 What Bdsyn Is

Bdsyn is  a tool  for quickly  describing and  implementing

combinational logic.    It allows  the user  to describe  a

circuit function  and  produce an  implementation  of  that

circuit automatically.     Bdsyn's output  is  given  in  a

"multiple-level" format.  This means that bdsyn is  capable

of translating  many  types of  logic  configurations  that

would  be impractical  to  represent  in  a  two-level  pla

form.  Bdsyn supports many high-level language  constructs:

subroutines, if--then--else,  select, constant bounded  for

loops,  multiple  assignment  to  variable,  and   multiple

returns  from  a  single  routine.    In  addition,   bdsyn

supports complex operators such as addition, and shifts  by

1

a variable amount.  (Note that complex operators may  imply

a large number of gates.)

When bdsyn  is used in  conjunction with mis,  it can  be

used to describe  logic in  standard cell, pla,  or a  more

complicated module generated form.

1.2 What Bdsyn Is Not

It is  very important to  keep in  mind that  bdsyn can  be

used for describing  combinational logic blocks  only.   We

define combinational logic to  be any block of logic  which

does not use system clocks or signal latching of any  kind.

Combinational logic does  not hold state  information.   In

particular,  describing a  finite  state machine  in  bdsyn

requires the logic designer (or a program) to add  external

latches to  the described  combinational logic  block  that

hold the current state of the machine.

The limitation  of  describing only  combinational  logic

is common  to pla description  programs as  well as  bdsyn.

(Although there may be provisions for automatically  adding

the required  external latches.)    The reason  we find  it

necessary to describe this limitation in such detail  here,

however, is that  some of the bdsyn  constructs (e.g.   for

loops and  multiple assignment) appear  very sequential  in

nature.   In  reality,  they are  merely interpreted  as  a

short hand for  describing parallel structure.   There  are

many examples  of the use  of sequential-like  descriptions

in the last section of this guide.

2 Using Bdsyn

2.1 Describing Logic in Bdsyn

Bdsyn descriptions are very similar to standard  procedural

programming languages (Pascal, C,  etc.)  Describing  logic

in bdsyn  amounts to  writing a  program that  accomplishes

the desired function.   Each flow-of-control structure  and

operation corresponds roughly  to a small  block of  logic.

Each variable  corresponds  to  "bit vector,"  which  is  a

group of single bit signals in the hardware.

At the top  of the bdsyn description  it is necessary  to

define input and  output "ports."   In a programming  point

of view,  these ports can  be viewed  as external  variable

declarations.   In a  hardware point of  view, these  ports

represent primary  input and outputs  of the  combinational

logic block.    Once  the  ports are  declared,  the  logic

designer should define a set of values for the input  ports

for which  he  wishes  the hardware  to  operate.    If  he

then writes  a program  that will  generate desired  output

2

values for  each  of the  interesting  input  combinations,

this constitutes a legal description.

To say it in a  different way, suppose a bdsyn  "program"

has been written.   Bdsyn will  create logic that will  act

in an  identical manner  to  that program.    If a  set  of

inputs were  to  be introduced  to  the bdsyn  program  and

the program  were executed  (run in  a software  sense),  a

particular set of  outputs would  be generated.   If  these

same inputs  were  introduced to  the  combinational  logic

block created  by  bdsyn, the  same  set of  outputs  would

result.

2.2 Running Bdsyn

Once  a hardware  description  has  been  written,  it  can

be  converted to  logic  by  running  bdsyn.     The  input

description is  read from  the file  given on  the  command

line.   The  blif output  is returned  to standard  output.

There are several command line options for bdsyn which  are

outlined in the  bdsyn manual page.   Two  of the  options,

however, will be discussed here.

The  -b  option  tells  bdsyn  not  to  do  its   cleanup

evaluation.       This  cleanup   evaluation   process   in

bdsyn   eliminates   redundant   and   unnecessary   logic.

Unfortunately,  it  may  also  have  the  undesired  effect

of eliminating user  specified intermediate variables  that

bdsyn considers unimportant.   The -b  makes sure that  the

variable are preserved, at the price of extra logic.1

The  -c  option is  followed  by  a  number  which  tells

bdsyn how  much "collapsing" of  logic that  it should  do.

Collapsing is  done  in order  to  decrease the  amount  of

output which  bdsyn  creates.    Although  quite  rare,  it

is possible  to create  an input  description which  causes

bdsyn to  collapse too much  logic and  run for  inordinate

amounts of time.  The  option -c1 will limit the amount  of

collapsing that is  done, and  will alleviate the  problem.

Bdsyn uses mis do to logic  collapsing for it.  The  actual

communication with mis is  accomplished through the use  of

unix pipes.  On non-unix  systems or on a system where  mis

is not available,  the option  -c0 should be  used to  turn

off collapsing altogether.

2.3 What is New in Version 1.1

There are  two  main changes  in  bdsyn since  version  1.0

that users  may  notice.   The  first  change is  that  the

intermediate variable names created  by bdsyn are  shorter.

Each variable  is  followed by  two  numbers.    The  first

------------------------

1The extra logic could easily be reduced by MIS at a later

time.

3

number  is the  block  in  the  description  in  which  the

variable is set.    The second number  represents how  many

times the variable has been revised in the block.

The second  change  is that  bdsyn  will create  its  own

intermediate  variables  corresponding  to  routine  return

values  and  condition  expressions  from  if  and   select

statements.    These  variables  are  good  candidates  for

having a large fanout.   They are created in the hope  that

it will save  mis some work  in extracting common  factors.

On occasion, bdsyn  may create  an internal variable  which

does not  fan out.    mis  will give  a warning  when  such

a  variable is  created.     In  general,  it  is  safe  to

ignore these warnings for  variables whose name begin  with

"$$COND."

3 Language Constructs

3.1 Names and Numbers

Bdsyn accepts  a  subset  of the  bds  language  as  input.

bds is  a Pascal-like  language.    The input  consists  of

names, numbers,  and reserved keywords  separated by  white

space.  White space is spaces, tabs, and carriage  returns.

Any characters  following a `!'    (exclamation point)  are

interpreted as comments by bdsyn.  Legal names may  contain

the characters  a-z, A-Z, 0-9,  `'  (underscore), and  `%'.

Other characters  are legal  in  the name  as long  as  the

name is quoted with  double quotes (").   Names may not  be

any of the reserved keywords (given below) unless they  are

quoted.   Names that begin  with `$' are  illegal.   Bdsyn,

like decsim, is case  insensitive, so the name "signal"  is

the same as the name "SIGNAL."

Numbers in  bdsyn must be  positive and may  be given  in

any one  of base  2, 4,  8, 10,  or  16.   Decimal  numbers

are represented as  a simple integer  (e.g.   511).   Other

base numbers are  represented as an  integer followed by  a

`#' and the base (e.g.   1FF#16).   The width, in bits,  of

a decimal  number is the  minimum number  of bits  required

to represent that  number.   The width of  a number with  a

`#' is equal  to the number of  bits required to  represent

a number  in that  base with  the given  number of  digits.

For example 001#2  has width three,  123#8 has width  nine,

and 12345678#16  has  width  thirty-two.    Bdsyn  has  the

limitation that  numbers in  the input  must be  thirty-two

bits or fewer.  This limitation may be overcome by the  use

of the concatenation operator.  (e.g.  12#16 &  34567890#16

is a forty bit constant.)

4

and       begin        buf            by       constant

do       downto       else          end       endmodel

endroutine endselect  endselectall  endselectone    eql

eqv        for         from          geq         gtr

if       leave        leq           lss        macro

mod       model        nand          neq         nor

not         or      otherwise        oxt       require

return    routine      select      selectall   selectone

sl0        sl1         slr           sr0         sr1

srr       state        sxt         synonym       then

to       width        xor           zxt

Table 1:  Bdsyn Reserved Words

behavior  endbehavior  endmodule entry  forward

fourstate     halt       inform   input  invalid

module     noentry     output    port  repeat

revision    static     twostate  until   while

Table 2:  Keywords Ignored by Bdsyn

3.2 Reserved Keywords

Table 1 gives a  list of reserved words  for bdsyn.   These

may not  be used  as  names unless  they are  quoted.    In

addition to the reserved  words there are several  keywords

which bdsyn  explicitly ignores  but are  included so  that

complete bds  descriptions  (not just  our subset)  may  be

parsed.  They are listed in table 2

3.3 Variables

Bdsyn has  two  types of  variables which  it  understands.

The first, "logic  variables," imply  actual logic and  can

be thought  of  as  wires in  the  actual layout.     Logic

variables are  declared  with bit  subscripts  that  define

their width.  The lower bit numbers correspond to the  less

significant bits, and bdsyn  requires that the high bit  be

first.

STATE name1<7:0>;

If  the  bit  range  of  a  variable  is  omitted  in   its

declaration,

STATE name2;

bdsyn will assume a single bit vector (name2<0>).

5

When using  a logic  variable (as  opposed to  specifying

one), the  bit subscript  is used  to choose  certain  bits

from the bit vector.   If the bit  vector is left out,  the

the entire bit vector is used.

In addition  to  logic variables,  bdsyn  defines  "meta-

variables."    Their  main  application  is  as  the  index

variable for for loops.  Loop index variables are  required

to be  meta-variables.   Meta-variables  are declared  with

null bit subscripts:

STATE name3<>;

Meta-variables  do  not  imply  any logic,   and  are  used

for temporary storage  of constants.   When  meta-variables

appear on the  left hand side of  an assignment, the  right

hand side is evaluated  and assigned to the  meta-variable.

The  right  hand  side   of  this  assignment  must  be   a

meta-variable expression (i.e., an expression written  from

meta-variables and constants).  When meta-variables  appear

on the  right  hand  of an  assignment,  they  are  treated

exactly like  constants.   This  means that  meta-variables

may be used in places where logic variables are  disallowed

by bdsyn.  Section 6 covers more about meta-variables.

3.4 Input Format

The following  is  a bnf  description  of a  typical  input

program for bdsyn.  Keywords are given in capital  letters,

and user names and productions are given in small  letters.

Optional  phrases are  given  in  square  brackets  `[  ]'.

Phrases in  curly braces  `{ }'  many be  repeated zero  or

more times in the input.  The character `|' (vertical  bar)

represents a choice of many lines.

The basic bdsyn input file consists of a "model":

MODEL model_name [output_var {, output_var}] =

[input_var {,  input_var}]

;

{global_declaration ;}

{routine}

ENDMODEL [model_name];

A "globaldeclaration" is defined as the following:

|   STATE global_var {, global_var}

|   CONSTANT constant_name = number {, constant_name  =

number}

|     SYNONYM  var_name  = var_name  {,  var_name  =

var_name}

A "routine" is defined as the following:

6

ROUTINE routine_name [bit_subscripts] [( var_param  {,

var_param} )] ;

{ local_declaration ;}

{ [ label_name: ] statement ;}

ENDROUTINE [routine_name];

A "localdeclaration" is defined as the following:

|   STATE local_var {, local_var}

|   CONSTANT constant_name = number {, constant_name  =

number}

|     SYNONYM  var_name  = var_name  {,  var_name  =

var_name}

A statement of a  routine is defined below.   Note:   The

square brackets in the  select statements represent  actual

brackets in the input.

|   BEGIN {statement ;} END

|   variable = expression

|   IF expression THEN statement [ELSE statement]

|   FOR meta_var FROM const_expr TO const_expr

[BY const_expr] DO statement

|   FOR meta_var FROM const_expr DOWNTO const_expr

[BY const_expr] DO statement

|       SELECT  expression  FROM  {[expression   {,

expression}]: statement;}

{[OTHERWISE]: statement  ;}

ENDSELECT

|      SELECTALL  expression  FROM  {[expression  {,

expression}]: statement;}

{[OTHERWISE]: statement  ;}

ENDSELECTALL

|      SELECTONE  expression  FROM  {[expression  {,

expression}]: statement;}

{[OTHERWISE]: statement  ;}

ENDSELECTONE

|   RETURN [expression]

|   LEAVE label_name

Note  that  these  tables   imply  that  a  semicolon   `;'

separates  every  statement.    As  in  Pascal,   there  is

no  semicolon before  else,  but  unlike  Pascal  there  is

a  semicolon before  end.     Expressions and  const-expr's

(constant  expressions)   will   be  described   in   later

sections.

7

3.5 Declaration Statements

The model statement defines the primary inputs and  outputs

to the  combinational logic  block being  described.    The

declared "ports" may not be meta-variables.

The state, constant,  and synonym may  be used as  either

global declarations,  applying to  all routines,  or  local

declarations, applying  to only  one routine.    The  state

statement is for declaration  of variables.  All  variables

must be  declared prior to  use.   Local  variables can  be

used only within  the scope of a  routine and their  values

are lost between consecutive calls to a routine.   Constant

declarations are  used  to assign  a  name to  a  constant.

Declared constants  can be  used anywhere  a constant  can.

Synonyms  are used  to  create  an  alternate  name  for  a

previously defined variable.  They are particularly  useful

for defining subfields of a variable:

SYNONYM opcode<6:0> = instruction<31:25>;

3.6 Flow-of-control Statements

Bdsyn   offers  a   rich   selection   of   flow-of-control

statements.   They are  if, select,  selectone,  selectall,

for,  return,  and leave.    The  if  statement  implements

optional  execution  of  a   statement.     If  the   least

significant bit  of  the if  expression evaluates  to  one,

the then  clause is  used.   If  the low  bit evaluates  to

zero, the else  clause (if present)  is used.   The  select

statement is  used to execute  one statement  out of  many.

If the  expression following  the select  keyword is  equal

to the  expression appearing  in the square  brackets,  the

corresponding statement is executed.

selectone  and  selectall  are  simply  a  shorthand  for

several  if  statements.     selectone  implies  nested  if

statements:

SELECTONE expr FROM

[expr1, expr2]: statement1;

[expr3]: statement2;

[OTHERWISE]: statement3;

ENDSELECTONE;

is equivalent to:

IF (expr EQL expr1) OR (expr EQL expr2) THEN

statement1

ELSE IF expr EQL expr3 THEN

statement2

ELSE

statement3;

8

selectall is equivalent to sequential if statements:

SELECTALL expr FROM

[expr1, expr2]: statement1;

[expr3]: statement2;

[OTHERWISE]: statement3;

ENDSELECTALL;

is equivalent to:

IF (expr EQL expr1) OR (expr EQL expr2) THEN

statement1;

IF expr EQL expr3 THEN

statement2;

IF (expr NEQ expr1) AND (expr NEQ expr2) AND (expr  NEQ

expr3) THEN

statement3;

selectall  is  different  than  select  in  that  selectall

implies sequentiality in its  case evaluation.  The  select

statement  can  be  viewed  as  having  all  of  its  cases

evaluated in parallel.    A selectall allows  two cases  to

evaluate to  true.   If  two cases  are true  in a  select,

incorrect logic may be produced.  select generally  implies

much less  logic,  however,  because there  is  no  implied

ordering of the cases.

for statements  are used  to iterate  a single  piece  of

code several times.   As  a hardware description  language,

bdsyn must  enforce  several rules  on the  for  statement.

The expressions  which specify  the  start, end,  and  step

values for  the loop  index must  be either  constants,  or

constant expressions.     This is  because  the  number  of

iterations must be  fixed.   Bdsyn also  requires that  the

index variable  of the  for statement  is a  meta-variable.

for loops  may  be  nested, and  since  meta-variables  are

treated as constants  by bdsyn, the  inner loop may  depend

on the outer loop.

FOR i FROM 0 TO 7 DO

FOR j FROM i TO 7 DO

statement;

The  return  statement  is   used  to  break  out  of   a

subroutine and  return to  the  calling routine.    If  the

subroutine returns a value (as described below) the  return

statement must be  followed by an  expression which is  the

return value.  The leave statement is similar to return  in

that can be  used to break  out of a  block of  statements.

When a leave statement  is encountered, control will  break

out of a labeled  statement.   The leave must occur  within

the labeled statement to which it points.  For example:

9

loop: FOR i FROM 1 TO 10 DO BEGIN

statement1;

IF condition THEN LEAVE loop;

statement2;

END;

Notice that by using the  leave it is possible to  describe

a loop  whose  number  of iterations  depends  on  a  logic

variable (condition in  this case).   It is only  important

that  the for  statement  itself  have  extents  which  are

constants or meta-variables.

3.7 Routines and Routine Calls

Bdsyn has  no  concept of  a  main routine,  per  se.    At

runtime, bdsyn  routines are  classified as  main  routines

or subroutines depending on  whether or not they called  by

another routine.   Bdsyn allows many  main routines in  the

same model,  and these routines  can be  viewed as  running

in parallel.   It  is an  error in bdsyn  for two  disjoint

routines (one is not called  by the other) to set the  same

global variable  or  primary output  of  the block.     Two

disjoint routines may  call the  same subroutine,  however.

It is illegal to have recursive routine calls.

Subroutines may  return  a single  variable  (regular  or

meta) to the  calling routine.   A subroutine that  returns

a value  is  declared  by placing  a  bit  subscript  after

the routine  name in  the  routine declaration.    The  bit

subscript specifies  the width of  the return  value or  if

the  value is  a  meta-variable.     If  the  subscript  is

missing, the routine returns  no value.  Parameters  passed

to subroutines  must be  declared as  part of  the  routine

declaration.  Parameter  passing has the same semantics  as

assignment statements (described below).

A common error in bdsyn is to have a subroutine which  is

not called by another routine.  This causes bdsyn to  treat

the subroutine as a main routine.

3.8 Assignment Statements

Assignment statements are written in the form:

variable = expression

They result  is that  the variable  receives the  value  of

the expression.   If  the width  of the  variable is  wider

than the  width of  the evaluated expression,  extra  zeros

will be  added  to  pad  the high  bits  of  the  variable.

If the  expression is  wider than  the variable,  the  high

bits are truncated.    If the variable  is a  meta-variable

10

the  expression  may  contain  only  constants  and   other

meta-variables.

It is  possible to assign  a value to  the same  variable

twice  in  the  same  program.     Bdsyn  will  treat  this

situation in the same manner as any sequential  programming

language.  In particular the following works correctly.

x = 1;   y = x;   x = 0;

IF x NEQ y THEN is_ok = 1;

Section 4 contains further details on multiple assignment.

3.9 Expressions

Legal expressions  are of  the following  form.   They  are

listed in order  of precedence,  and expressions higher  on

the list  are evaluated first.    The curly  braces in  the

zxt, oxt, and  sxt expressions represent actual  characters

in the input, not a repeatable phrase.

|   number                                ! number  as

described above

|   constant                             ! a  declared

constant

|   meta_variable

|   regular_variable

|    (expression)                      !  parenthesized

expression

|   expression<expression>          ! choose a bit

|    expression<const_expr:const_expr>        !  choose

several bits

|   expression & expression         ! concatenation

|    expression SR0  expression         ! shift  right

filling with zeros

|    expression SR1  expression         ! shift  right

filling with ones

|   expression SRR expression       ! rotate right

|    expression  SL0 expression          ! shift  left

filling with zeros

|    expression  SL1 expression          ! shift  left

filling with ones

|   expression SLR expression       ! rotate left

|   expression * expression         ! multiplication

|   const_expr / const_expr         ! integer division

|   const_expr MOD const_expr       ! mod operation

|   expression + expression         ! addition

|   expression - expression         ! subtraction

|   expression EQL expression       ! Equal comparison

|     expression NEQ  expression          ! Not  equal

comparison

11

|     expression LSS  expression          ! Less  than

comparison

|   expression LEQ  expression       ! Less than  equal

comparison

|    expression GTR expression         ! Greater  than

comparison

|    expression GEQ expression         ! Greater  than

equal comparison

|   BUF expression                  ! null operation

|   NOT expression                  ! bitwise NOT

|   expression AND expression       ! bitwise AND

|   expression NAND expression      ! bitwise NAND

|   expression OR expression        ! bitwise OR

|   expression NOR expression       ! bitwise NOR

|   expression XOR expression       ! bitwise XOR

|   expression EQV expression       ! bitwise XNOR

|   WIDTH expression                ! return number  of

bits

|    ZXT {WIDTH =  const_expr} expression      !  zero

extend

|    OXT {WIDTH  = const_expr} expression       !  one

extend

|    SXT {WIDTH =  const_expr} expression      !  sign

extend

In the above "constexpr" represents a constant  expression.

A constant expression is  an expression that contains  only

numbers, declared constants,  and meta-variables.   It  may

not contain regular variables.

The single  bit  selection operator,  <0>,  may  take  an

arbitrary expression as  the bit  selector.   When the  bit

selector is  not  a  constant, a  multiplexor  is  implied.

The multiple bit selection operator, <31:0>, requires  that

both expressions  be  a constant  expression and  that  the

first number is larger than the second.  The  concatenation

operator  `&'  joins  two  bit  vectors  into  one.     The

expression before the `&' becomes the high order bits,  the

trailing expression becomes the low order bits.

The shift  operations shift  the first  expression by  an

amount given  by the  second expression.    When the  shift

amount  is a  variable  expression,  a  barrel  shifter  is

thelwidth  ofothe shiftedshexpression.on Thiss means itanis

possible to  shift  bits  off the  end  of  the  expression

accidentally.     Care  should  be  taken  to  extend   the

expression (using zxt) so no bits are lost.

`+', `-', `*', `/', and mod implement integer  arithmetic

and are supported for  constant expressions.  In  addition,

`+', `-', and `*'  are supported for variable  expressions.

`+' and `-' imply a  combinational adder in this case,  and

12

`*' implies  a  full combinational  multiplier.    The  `*'

operation should be used sparingly on variable  expressions

since it implies so much logic.  In particular, `*'  should

not be used where a shift would be sufficient.

The comparison operators  evaluate to a  single bit 1  if

the condition is true and  a single bit 0 if the  condition

is  false.    If  an  expression in  the  comparison  is  a

variable, a  comparator is implied.    The bitwise  boolean

operators take  two  bit vectors  and perform  the  desired

operation bit  by  bit.    If  one of  the  expressions  is

smaller than  the  other,  it will  be  automatically  zero

extended.  The width  operator returns a constant which  is

the number of bits in the given expression.  zxt, oxt,  and

sxt will add more significant bits to the given  expression

to make it the  given width.  It  is an error to specify  a

width that is smaller than the given expression.

3.10 Macro Definitions and Required Files

Bdsyn  has the  facility  for  defining  macro  definitions

in the  input description.   Macro  definitions may  appear

anywhere in the  input description and  have the  following

format:

MACRO macro_name = macro_body $ENDMACRO

Following a  macro definition, any  use of  macroname  will

be replaced by the  macrobody.   The macrobody is  expanded

exactly  as typed  (including  comments).     It  may  span

more than  one line  and may  contain uses  of other  macro

definitions.

Bdsyn also  allows distinct  input files  to be  included

within other  files.     This is  done  using  the  require

command.  A require may occur anywhere in a bdsyn file  and

has the syntax:

REQUIRE 'filename.ext';

The text of the required file will be inserted in place  of

the require command.

4 Multiple Assignment

As   mentioned  in   the   introduction,    bdsyn's   input

descriptions  appear very  sequential  in  nature  yet  are

intended to  describe combinational logic.    We feel  that

our form of sequential  description gives the user a  great

deal of power  and flexibility  in specifying logic.    The

translation  of  sequential  description  to  combinational

logic  is  accomplished  by  correctly  handling  "multiple

assignment."

13

Our definition of multiple assignment is the correct  and

consistent handling of many  sequential assignments to  the

same variable.  For example:

x = default_value;

IF condition THEN x = new_value;

describes a  multiplexer  which is  controlled  by  "condi-

tion."   The above  example could  be rewritten  in a  more

conventional way:

IF condition THEN

x = new_value

ELSE

x = default_value;

At first glance, one  might think that multiple  assignment

is not particularly useful.   In fact, multiple  assignment

is the  heart  of  bdsyn's ability  to  process  sequential

descriptions.   Several examples  of this can  be found  in

the "Examples" section at the end of this guide.

5 Unspecified Variables

In  bdsyn  input  descriptions,   as  in  any   programming

language, it is  possible to have  variables which are  not

always defined.  For instance, in the following code:

IF cond THEN

x = expr;

the variable x, if it has not previously been assigned  to,

is unspecified when cond is false.

Under normal  circumstances bdsyn will  assume that  when

variables  are unspecified,  they  should  have  the  value

zero.   This assumption  is probably not  a good one.    In

some cases, the  unspecified variables  could be caused  by

a mistake in the  user specification.   In other cases  the

user may  not care  about a  variable value  under  certain

conditions.  In this case the user should probably  specify

DONTCARE conditions as discussed in section 8.

In general,  it is  very difficult  for  bdsyn to  detect

the use  of unspecified  variables.   However,  unspecified

variables may  be detected  by  using mis.    First,  bdsyn

should be run using the -n flag.  This will cause bdsyn  to

create a group  of variables that  end with the  characters

"**".  (Also,  the ".inputs" line  in the blif output  will

not be  printed.)     The blif  file  containing  the  "**"

variables should  then  be  read into  mis  and  minimized.

After  minimization,  if any  of  the  "**"  variables  fan

out to  any  gates,  this  corresponds to  the  use  of  an

unspecified variable.

14

6 Meta-variables

Meta-variables  have  a  very   separate  use  from   logic

variables.   They can be  used within constant  expressions

because they  represent  constant  values.    Unlike  logic

variables,  they  are  guaranteed  to  always  assume   the

same particular  value.    The value  of a  logic  variable

may  change based  on  the  primary  inputs  to  the  bdsyn

combinational block.

Because bdsyn is  able to  statically evaluate the  value

of  meta-variables,  it is  able  to  perform  some  simple

logic minimization when  meta-variables are involved.    In

the following example, function1 and function2 are  complex

functions.  (i is a meta-variable.)

FOR i FOR 0 TO 5 DO

IF i MOD 2 EQL 0 THEN

x = function1(x)

ELSE

x = function2(x);

After bdsyn minimization this becomes:

x = function1(x);

x = function2(x);

x = function1(x);

x = function2(x);

x = function1(x);

x = function2(x);

Had the  condition in the  if statement  above depended  on

a logic  variable,  no such  minimization would  have  been

possible inside of bdsyn.   This is because the value of  a

logic variable is generally unpredictable.

One  non-obvious  problem  with  meta-variables  is   the

following.  A  meta-variable is defined within a  branching

structure (i.e.   if, select, etc.).   If the condition  of

the branching structure  depends on a  logic variable,  the

meta-variable is not correctly defined upon exit from  that

structure.  For example:

meta = 3;

IF x EQL 1 THEN BEGIN

meta = 4;

a<0> = w<meta>;     ! This works correctly

END

a<1> = w<meta>;         ! This uses meta = 4 even if  (

x NEQ 1 )

This problem  stems from  indirectly trying  to define  the

meta-variable meta in terms of  the logic variable x.   For

similar reasons,  meta-variables  that  are used  for  loop

counts are  not  necessarily  correct upon  exit  from  the

loop.  For example:

15

loop: FOR i FROM 1 TO 10 DO BEGIN

x = function(x);

IF x EQL 1 THEN LEAVE loop;

END;

y = i;                ! y gets the value 10  regardless

of the function

7 Complex Operators

Bdsyn  supports  several  operations   which  can  not   be

implemented by  just  one or  two  gates.    These  include

`+',  `-',   `*',   leq,  geq,   lss,  gtr,   shifts,   and

single  bit selection  when  they  are  operating  on  non-

constants.   The operations imply  complex structures  such

as adders, multipliers,  comparators, barrel shifters,  and

multiplexors.     The  complex  operators  are  implemented

through  a set  of  library  routines  found  in  the  file

"bdsyn.lib".  The macros in the library are used to  create

routines of an appropriate size (a 3 bit adder, or a 4  bit

comparator, etc) which are then inserted into the code.

8 Don't Care Conditions

It is often the case  that it makes no difference what  the

value of  an output is.   For  example, in  a finite  state

machine with  seven  states encoded  in three  state  bits,

state 7 never occurs:

SELECT state<2:0> FROM

[0,1,2,3]: output = 0;

[4,5,6]: output = 1;

ENDSELECT;

Here, the  output for  state 7 would  be unspecified.    To

resolve this, a default value for output might be added.

output = 0;

SELECT state<2:0> FROM

[0,1,2,3]: output = 0;

[4,5,6]: output = 1;

ENDSELECT;

The problem  is  that this  is  unnecessarily  restrictive,

and  does  not  allow  for  all  of  the  potential   logic

simplification.   In actuality, it  may make no  difference

whether "output = 0"  or "output = 1"  in state 7,  because

state  7  never  occurs.    For  this  reason,   bdsyn  has

the special  variable  DONTCARE,  which  will  allow  logic

optimization tools to choose the optimal value for  output.

DONTCARE is used as any other variable:

16

output = DONT_CARE;

SELECT state<2:0> FROM

[0,1,2,3]: output = 0;

[4,5,6]: output = 1;

ENDSELECT;

Inside  of  bdsyn,  assignments  from  this  variable  will

automatically  be  extended   to  fit  the   size  of   the

destination.  When DONTCARE is used, it will  automatically

be added to  the input  port list in  bdsyn's blif  output.

There is no need to declare the DONTCARE variable.

The current  version of  mis can  not process  the  don't

care conditions produced by bdsyn.   This is because  don't

care specifications in multiple  level descriptions can  be

ambiguous in their  meaning.   Until a  uniform method  for

handling multiple level don't  cares is implemented, it  is

still possible to use espresso  to make partial use of  the

don't care specification.  This can be accomplished by  the

following (in unix):

bdsyn input_descr | mis -c clp -T pla | \

espresso -Dmapdc | espresso > output.pla

The resulting pla can then be read into mis.

If  a  description  has  been  written  that  includes  a

don't care  specification and  the use  of the  don't  care

specification is  not  desired,  the -z  option  should  be

used.    The  -z  option maps  every  use  of DONT-CARE  to

zero.  The effect will be that in places where DONTCARE  is

specified, the result will be forced to the value zero.

9 Examples

Figures  1 through  5  contain  five  examples  of  various

bdsyn  descriptions.     The  first  is  a  generic  finite

state machine.    It merely  generates the  next state  and

control signals  based on  the  present state.    The  next

three  descriptions  implement  the  exact  same  piece  of

logic.   They are included  to demonstrate the  flexibility

of bdsyn.   Note  that the  second implementation  (COUNT2)

uses intermediate  variables seenZero  and seenTrailing  as

state holders.  The  variables do not really imply  latches

to hold the  state.   They are simply  a convenient way  to

describe an idea.

The  fifth example  is  a  complicated  decoder  circuit.

Particular attention should be given to the last  statement

of bdsyn.in TheudecoderItisidescribedsasn ambarrelt shifter

which shifts a  single bit  1 onto the  correct word  line.

This  is obviously  a  very  inefficient  way  to  build  a

17

decoder.   It  is important to  note, however,  that  bdsyn

and mis  are capable  of reducing  the hardware  to a  more

sensible implementation.    There is  generally no  penalty

associated with  a quick  and dirty  shortcut  description.

The logic that is generated is the same.

10 Acknowledgments

The parser, inline  routine expansion, and macro  expansion

inside  of  bdsyn  were  implemented  by  Richard   Rudell.

The  for  loops,  return  and leave  processing,   multiple

assignment,  and  mapping  to  blif  were  implemented   by

Russell Segal.   To  do logic collapsing,  bdsyn uses  mis,

which was implemented  by Albert Wang  and Richard  Rudell.

We  would  like  to  thank  the  students  and   industrial

visitors who  participated  in the  Spring  1986  synthesis

class for  running  bdsyn  through  its paces.     We  also

would like  to  thank Professors  Richard  Newton,  Alberto

Sangiovanni-Vincentelli, and  Carlo Sequin  for  organizing

the synthesis project and given bdsyn a reason to exist.

18

!  The  classic  traffic  light  controller  finite   state

machine from

! Mead and  Conway, Introduction to  VLSI Technology,  page

85

MODEL traffic_light

hl<1:0>,            ! control for highway light

fl<1:0>,            ! control for farm light

st<0>,              ! to start the interval timer

nextState<1:0> =

c<0>,                  !  indicating a car on the  farm

road

ts<0>,              ! timeout of short interval timer

tl<0>,              ! timeout of long interval timer

presentState<1:0> ;

! state assignments

CONSTANT HG = 0, HY = 2, FG = 3, FY = 1;

! symbolic output assignments

CONSTANT GREEN = 0, RED = 1, YELLOW = 2;

ROUTINE traffic_light_controller;

! set up default outputs (use of multiple assignment)

nextState = presentState;

st = 0;

SELECT presentState FROM

[HG]: BEGIN

hl = GREEN;   fl = RED;

IF c AND tl THEN BEGIN

nextState = HY;

st = 1;

END;

[HY]:;BEGIN

hl = YELLOW;   fl = RED;

IF ts THEN BEGIN

nextState = FG;

st = 1;

END;

END;

[FG]: BEGIN

hl = RED;   fl = GREEN;

IF NOT c or tl THEN BEGIN

nextState = FY;

st = 1;

END;

[FY]:;BEGIN

hl = RED;   fl = YELLOW;

IF ts THEN BEGIN

nextState = HG;

st = 1;

END;

END;

ENDSELECT;

ENDROUTINE;                 19

ENDMODEL;

Figure 1:  Traffic Light Controller

! COUNT1 (Count Zeros):

! The input is an  8 bit binary string expected to  consist

of

! a string of  1's, a string of 0's,  and a string of  1's.

Any

! of these strings may be zero length.

! For example, `11000111',  `10000001', and `11110000'  are

all

! valid strings, and `10001100' is a mal-formed string.

! The problem  is to design  a circuit which,  given the  8

bit

! string, returns the count of number of consecutive  zeros

in

! the string, or returns  an error condition if the  string

is

! mal-formed.  The output is undefined if the input  string

is

! mal-formed.

MODEL count1 error<0>, out<3:0> = in<7:0>;

! find first bit matching 'val' (return index of the bit)

ROUTINE ff<3:0>(x<7:0>, val<0>);

STATE i<>;

FOR i FROM 0 TO 7 DO

IF x<i> EQL val THEN

RETURN i;

RETURN 8;

ENDROUTINE;

ROUTINE main;

STATE x<7:0>;

! Shift off the first string of 1's

x = in SR1 ff(in, 0);

! Count is where the first 1 is

out = ff(x, 1);

! Check for a mal-formed string:

! shift off 0's, and check for any more 0's

x = x SR1 out;

error = ff(x, 0) NEQ 8;

IF error THEN

out = DONT_CARE;

ENDROUTINE;

ENDMODEL;

Figure 2:  Count1

20

! COUNT2 (Count Zeros)

! A different implementation for the last example.

MODEL count2 error<0>, out<3:0> = in<7:0>;

CONSTANT TRUE = 1, FALSE = 0;

ROUTINE legal<0>(x<7:0>);

STATE i<>;

STATE seenZero;         ! there has been a zero

STATE seenTrailing;       ! we  have hit the  trailing

ones field

seenZero = FALSE;

seenTrailing = FALSE;

FOR i FROM 0 TO 7 DO

IF seenTrailing AND (x<i> EQL 0) THEN

RETURN FALSE                         !  Illegal

string

ELSE IF seenZero AND (x<i> EQL 1) THEN

seenTrailing = TRUE

ELSE IF x<i> EQL 0 THEN

seenZero = TRUE;

! If we made it to here then the input is legal

RETURN TRUE;

ENDROUTINE;

ROUTINE zeros<3:0>(x<7:0>);

STATE i<>, count<3:0>;

count = 0;

FOR i FROM 0 TO 7 DO

IF x<i> EQL 0 THEN

count = count + 1;

RETURN count;

ENDROUTINE;

ROUTINE main;

IF legal(in) THEN BEGIN

error = FALSE;

out = zeros(in);

END ELSE BEGIN

error = TRUE;

out = DONT_CARE;

END;

ENDROUTINE;

ENDMODEL;

Figure 3:  Count2

21

! COUNT3 (Count Zeros)

! Yet another implementation

MODEL count3 error<0>, out<3:0> = in<7:0>;

CONSTANT TRUE = 1, FALSE = 0;

! check for a legal string

ROUTINE legal<0>(x<7:0>);

STATE i<>, j<>, k<>;

! Look for a ...0...1...0...

FOR i FROM 0 to 5 DO

FOR j FROM i+1 to 6 DO

FOR k FROM j+1 to 7 DO

IF (x<i> EQL 0) AND (x<j> EQL 1) AND

(x<k> EQL 0) THEN

! The input is illegal

RETURN FALSE;

! If we made it to here then the input is legal

RETURN TRUE;

ENDROUTINE;

ROUTINE zeros<3:0>(x<7:0>);

STATE i<>, count<3:0>;

count = 0;

FOR i FROM 0 TO 7 DO

IF x<i> EQL 0 THEN

count = count + 1;

RETURN count;

ENDROUTINE;

ROUTINE main;

IF legal(in) THEN BEGIN

error = FALSE;

out = zeros(in);

END ELSE BEGIN

error = TRUE;

out = DONT_CARE;

END;

ENDROUTINE;

ENDMODEL;

Figure 4:  Count3

22

! Register decoder for the SPUR cpu chip.

! The register file of the SPUR cpu uses a Berkeley RISC

! type register windowing scheme.  The register file is

! divided into 8 overlapping register "windows".  Each

! register window can access 32 of SPUR's 138 registers.

! The first 10 registers of all of the windows point to

! the same 10 "global" registers.  The next 6 registers

! are shared with the top six registers of the previous

!  window.    The  6  top  registers  of  each  window  are

similarly

! shared with the next window.  The last window wraps

! around and shares registers with the first window.

MODEL reg_decode

addr<137:0>     ! one-hot decoded output (word lines)

=

cwp<2:0>,       ! Current window pointer

reg<4:0>;       ! Current register in the window

CONSTANT

NUMREGS = 138,      ! total number of registers

NUMGLOBALS = 10;    ! number of global registers

ROUTINE REG_DECODE();

STATE

sum<7:0>,       ! temporary variable

decAddr<7:0>;    ! decoded address (number of  the

word line)

! calculate the correct register number

IF (reg LSS NUMGLOBALS) THEN

! point to the global registers

decAddr = regSpec

ELSE BEGIN

sum = (cwp & 0000#2) + regSpec;

! Check for wrap around.  If so, need to  subtract

an offset

IF (sum GTR (NUMREGS - 1)) THEN

sum = sum - (NUMREGS - NUMGLOBALS);

decAddr = sum;

END;

!  Do the  actual  decoding (***  See main  text  for

details ***)

addr = (ZXT {width=138} 1) SL0 decAddr;

ENDROUTINE;

ENDMODEL;

Figure 5:  Register file decoder

23