HELP FOR CREATING A PALETTE FOR OCTOPUS

1.0 WHAT IS A PALETTE

1.1 What Does an Octopus Palette Do?

A palette is an OCT cell that contains instances of cells and other

information used by an application.  It contains all the technology- specific

information needed for creating a PLA layout, with the exception of design

and spacing rules for compaction.  This information falls into two categories:

instances of cells and wiring rules.

For octopus to store layout in the OCT database, the full path to the

master of each instance used must be known.  This information is stored

within the palette.  Cells used in octopus' layout include MOSFETs, input

and output buffers, contacts, and pads.

For octopus to use the mkarray (3CAD) package to wire all the instances

together, it needs rules to guide the wiring.  These rules are are described

in the OCT database, and are stored in the palette.  The rules tell mkarray

which terminals of which instances should be connected.  The application

(octopus) then uses mkarray to apply the wiring rule to a row, column, or area.

See mkarray documentation for a more complete description of pattern-based

wiring and the shape and content of a mkarray wiring rule, and palette.c for

examples of how to construct wiring rules in OCT.

Octopus uses a palette because it provides a level of indirection in

locating technology-specific data used in creating a PLA.  If a new palette

is provided, cells and rules from the new palette will be used and the

resulting PLA will be constructed accordingly.  This allows the user to

implement the PLA in different technologies or with different cell libraries

without having to change octopus.

1.2 How Does Octopus Use the Palette?

Each cell that octopus needs must be instantiated within the palette,

and each wiring rule needed by mkarray must be in the palette.  Octopus uses

OCT to access objects in the palette by name.  This means that each instance

in the palette must be given a unique name, and octopus must know that name.

For example, the default palette has an instance of a MOSFET called "andTran."

When octopus needs to place a transistor in the AND plane, it asks OCT to give

it a copy of the instance within the palette called "andTran."  Note that it

doesn't matter what the name of the master of the instance is; the requirement

is that the instance within the palette must be called "andTran."  If the

definitions of instance and master are not clear, see "The General Structure

of OCT."  Octopus instantiates into the AND plane whatever instance OCT found

with the name "andTran." If a new palette is provided with a different instance

named "andTran," then the master of that instance will used as the AND plane

transistor.  Similarly, each rule must be given a unique name and octopus

must know that name in order to access the rule.

2.0 WHAT IS IN AN OCTOPUS PALETTE AND HOW DO I MAKE ONE?

2.1 What Is In an Octopus Palette?

The most precise description of what is needed in an octopus palette

comes form examining an actual octopus palette, the octopus data structures

that store information about each rule and cell used, and palette.c, the

source code for the program we use to automatically create the default palette.

Octopus is still an evolving program, so exact details of what is currently

in the octopus palette are not too useful.  However, here is a general

description of the types of data in an octopus palette.

An octopus palette must contain the following:

(1)  An instance of each cell used in the layout.  For each cell

that octopus instantiates, there must be an instance of the

cell within the palette.  This instance must be named within

the palette, and octopus must know what that name is so the

instance can be retrieved from the palette by name.  The

instances include both types of transistors, the input and

output buffers (both clocked and not clocked), and all the

connector cells (contacts, pads, etc.).

(2)  Wiring rules.  These are used to instruct the mkarray package

on how to connect instances together.  Wiring rules associate

(singly, in pairs, or in groups) the actual terminals (of

instances) that should be wired together.  Mkarray then

determines the appropriate layer and path to connect the

terminals.

2.2 How Do I Make My Own Palette?

With the octopus source code comes palette.c, the source code for the key

procedures of the program used to construct the default palette.  The program

is called createPalette (i.e., that's the makefile target).  Palette.c is

a good place to start from when building a custom palette.

To keep track of the names, octopus and createPalette use two data

structures, defined in octopus.h, called s_cells and s_ruleNames.  Both these

data structures are initialized in defaults.c.  Since the names are needed

both by octopus and createPalette, the initialization procedure is shared by

both programs to insure consistency in the names used.

After initialization, createPalette instantiates each of the cells.  The

name for the instance and the path to the master of the instance are stored

in s_cells.  Both of these are used by createPalette, but octopus only uses

the instance name.  Octopus gets the path to the master from OCT (as it's

stored in the palette).

The next step is to create the wiring rules.  A wiring rule is a collection

of bags and terminals connected into a tree in OCT.  OCT bags form the internal

nodes of the tree, and OCT instance actual terminals are the leaves.  The

mkarray package interprets these trees and uses them to guide pattern-based

wiring.

To make your own palette, I recommend you follow this model.  Instantiate

the cells, then connect the appropriate terminals into mkarray wiring rule

trees.

2.3 Some Things to Remember

(1)  All the instance and rule names must be unique.

(2)  For different cell libraries, the number and locations of

terminals on instances may change enough necessitate change

to the code that activates wiring rules and makes the calls

to the mkarray wiring procedures.

(3)  If the names of the instances or the names of the rules are

changed in a new palette, the original distributed version

of octopus will not work with that palette.

Questions and comments to octopus@eros.Berkeley.EDU