Summary:

void   simplify_node();

node_t *simp_dc_filter();

typedef enum sim_method_enum sim_method_t;

enum sim_method_enum {

SIM_METHOD_DCSIMP, SIM_METHOD_NOCOMP, SIM_METHOD_SNOCOMP

};

typedef enum sim_dctype_enum sim_dctype_t;

enum sim_dctype_enum {

SIM_DCTYPE_NONE, SIM_DCTYPE_FANIN, SIM_DCTYPE_FANOUT,

SIM_DCTYPE_INOUT, SIM_DCTYPE_ALL, SIM_DCTYPE_SUB_FANIN,

SIM_DCTYPE_UNKNOWN

};

typedef enum sim_filter_enum sim_filter_t;

enum sim_filter_enum {

SIM_FILTER_NONE, SIM_FILTER_EXACT, SIM_FILTER_DISJ_SUPPORT,

SIM_FILTER_SIZE, SIM_FILTER_FDIST, SIM_FILTER_SDIST,

SIM_FILTER_ALL

};

void

simplify_node(f, method, dctype, accept, filter)

node_t *f;

sim_method_t method;

sim_dctype_t dctype;

sim_accept_t accept;

sim_filter_t filter;

Simplify f using "method", with "dctype" don't-care set

filtered using "filter", and accept the result using

"accept" criteria.

"method" is used to specify what algorithm is to be used

to simplify f.

SIM_METHOD_SNOCOMP is a single pass of NOCOMP (See next option).

SIM_METHOD_NOCOMP uses ESPRESSO with EXPAND being replaced

with a new algorithm.  See Abdul Malik's paper on ICCAD-88.

SIM_METHOD_DCSIMP uses reduced offset based ESPRESSO.

This is advantangous over regular espresso when the

off-set is too large.

"dctype" is used to specify what type of don't-care set is

to be generated.

SIM_DCTYPE_NONE generates no don't-care set.

SIM_DCTYPE_FANIN generates fanin don't-care sets.

SIM_DCTYPE_FANOUT generates one-level fanout don't-care set.

SIM_DCTYPE_INOUT combines SIM_DCTYPE_FANIN and SIM_DCTYPE_FANOUT.

SIM_DCTYPE_ALL generates all the don't-care set that we are

be able to generate currently.  This should be used

with caution for the don't-care set may be too large.

SIM_DCTYPE_SUB_FANIN generates fanin don't cares only for nodes

with the same or subset support as the node being

minimized. This is very effective for fast simplification

when the don't care sets are very large.

SIM_DCTYPE_NONE should not be used.

"filter" is used to trim the don't-care set to speed up the

minimization process.

SIM_FILTER_NONE doesn't trim the don't-care set at all.

SIM_FILTER_EXACT only trims out the absolutely un-necessary

part of the don't-care set.

SIM_FILTER_DISJ_SUPPORT removes all the cubes in the don't-care

set which have disjoint support from the support of the

on-set.

SIM_FILTER_SIZE removes all the cubes in the don't-care

set which are have more than a fixed number of literals (3).

SIM_FILTER_FDIST removes all the cubes in the don't-care

set which are more than a fixed number of literals outside

the support of the node being simplified (2).

SIM_FILTER_SDIST removes all the cubes in the don't-care

set which are more than a fixed distance from all the cubes

of the node being simplified (2).

"accept" is used to specify the acceptance criteria.

SIM_ACCEPT_FCT_LITS accepts the result if the number

of literals in the factored form decreases.

SIM_ACCEPT_SOP_LITS accepts the result if the number

of literals in the sum-of-product form decreases.

SIM_ACCEPT_CUBES accepts the result if the number of

cubes decreases.

SIM_ACCEPT_ALWAYS forces the acceptance.

void

comp_perm_fn(network, method, dctype, accept, filter)

network_t **network;

sim_method_t method;

sim_dctype_t dctype;

sim_accept_t accept;

sim_filter_t filter;

Simplify each node in the network using "method",

and accept the result using "accept" criteria.

The don't care sets are created and filtered internally.

"method" is used to specify what algorithm is to be used

to simplify f.

SIM_METHOD_SNOCOMP is a single pass of NOCOMP (See next option).

SIM_METHOD_NOCOMP uses ESPRESSO with EXPAND being replaced

with a new algorithm.  See Abdul Malik's paper on ICCAD-88.

SIM_METHOD_DCSIMP uses reduced offset based ESPRESSO.

This is advantangous over regular espresso when the

off-set is too large.

"dctype" is not used at this point. A subset of observability

and satisfiability don't cares are created internally for each

node. The options are exactly the same as simplify_node().

"filter" is not used at this point. All the filtering is done

internally. The options are exactly the same as simplify_node().

"accept" is used to specify the acceptance criteria.

SIM_ACCEPT_FCT_LITS accepts the result if the number

of literals in the factored form decreases.

SIM_ACCEPT_SOP_LITS accepts the result if the number

of literals in the sum-of-product form decreases.

SIM_ACCEPT_CUBES accepts the result if the number of

cubes decreases.

SIM_ACCEPT_ALWAYS forces the acceptance.

node_t *

simp_dc_filter(f, dc, filter);

node_t *f;

node_t *dc;

sim_filter_t filter;

Returns the don't-care set after applying the "filter".

"f" is the on-set, and "dc" is the don't-care set.