Designing Finite State Machines with PEG

_#G_#o_#r_#d_#o_#n _#H_#a_#m_#a_#c_#h_#i

Computer Science Division

Electrical Engineering and Computer Sciences

University of California

Berkeley, CA  94720

ABSTRACT

PEG is a finite state machine compiler.   It  translates

high   level   language  descriptions  of  finite  state

machines into the logic equations  needed  to  implement

state  machine designs.  Since the output format is com-

patible with _#e_#q_#n_#t_#o_#t_#t, PEG may be used as a front end for

Berkeley PLA tools.

_#1.  _#I_#n_#t_#r_#o_#d_#u_#c_#t_#i_#o_#n

_#P_#E_#G (PLA Equation Generator) is a  design  tool  for  finite

state  machines.  It compiles high level language descriptions of

finite state machines into the logic equations needed  to  imple-

ment a design.

_#P_#E_#G programs are isomorphic to Moore machine state diagrams.

There  is  a  one-to-one correspondence between states in a state

diagram and state definitions in the corresponding  _#P_#E_#G  program.

The translation from state diagrams to _#P_#E_#G programs is simple and

straightforward.

Designing with PEG provides a number of advantages over  the

traditional  pencil-and-paper  approach  method  of  FSM  design.

PEG's high level language enables designs and design  changes  to

quickly  be implemented.  PEG programs provide easy-to-understand

documentation with clear control flow.  PEG does the tedious  and

error-prone  bookkeeping task of generating _#o_#u_#t_#p_#u_#t and _#n_#e_#x_#t _#s_#t_#a_#t_#e

bits as a function of current state bits.  It checks  for  design

- 1 -

Designing Finite State Machines with PEG                  2/28/85

errors and eliminates redundant terms in logic equations.

As output PEG generates logic equations in  the  _#e_#q_#n  format

accepted  by  _#e_#q_#n_#t_#o_#t_#t [Cmelik], another Berkeley design tool.  By

piping the output of PEG through _#e_#q_#n_#t_#o_#t_#t, PEG may be  used  as  a

front  end  for  Berkeley  PLA  tools  such  as  _#m_#p_#l_#a [Mayo], and

_#e_#s_#p_#r_#e_#s_#s_#o [Rudell].   As  an  option,  _#P_#E_#G  will  also  print  the

unminimized  truth  table from which the logic equations are der-

rived.

_#2.  _#A _#S_#i_#m_#p_#l_#e _#E_#x_#a_#m_#p_#l_#e

Figure 1 shows the state diagram  for  a  four-state  finite

state  machine  implementing  a  2-bit  binary  counter.  The _#P_#E_#G

description of this design appears in Figure 2.  The program  has

no  inputs  besides  an implicit clock.  The outputs of the state

machine are its _#n_#e_#x_#t _#s_#t_#a_#t_#e bits,  which  are  automatically  gen-

erated by _#P_#E_#G.

Figure 1:  State Diagram for Example 1

In its most simple form, a _#P_#E_#G program consists of a list of

state  descriptions.   The  sample program has four states.  Each

state has four parts:  an optional label, a  colon,  an  optional

signal assertion part, and and optional control part.

--Simple PEG program for 2-bit counter

--State transition on every clock

--No reset => starts in a random state

Start         :              --This is state 0

:              --This is state 1

:              --This is state 2

:              --This is state 3

GOTO Start;

Figure 2:  _#P_#E_#G Program for Example 1

- 2 -

Designing Finite State Machines with PEG                  2/28/85

The  first  state  in  the  example  is  labeled  with   the

identifier  _#S_#t_#a_#r_#t.   The  label  is necessary only because of the

GOTO from state 3 back to state 0.

States 1 and 2 are examples of the  minimal  state  descrip-

tion.  These states are completely defined by a colon, which acts

as a place holder for the  state.   Empty  states,  in  which  no

branching  or  signal  assertions  occur,  are  sometimes used to

introduce necessary delays in FSMs.

Flow of control in _#P_#E_#G programs is sequential unless  other-

wise  specified.  Since  no  control  information  is present for

states 0, 1, and 2, the program steps  sequentially  through  the

states  0, 1, 2, and 3.  State 3 has control information specify-

ing a jump back to the state labeled _#S_#t_#a_#r_#t.

Since it has no sequential _#n_#e_#x_#t _#s_#t_#a_#t_#e, control  must  always

be  defined  for the last state in the program.  _#P_#E_#G generates an

error message and quits if control is not defined  for  the  last

state.

Although state transitions are performed on clock ticks,  no

clock is mentioned in the program.  It is the user's responsibil-

ity to implement the state  machine  with  synchronous  logic  to

latch input and output signals.

Comments begin with a double dash "--" and terminate at  the

end  of  the line on which they appear.  The first three lines of

the program are  comments.   Comments  also  appear  on  lines  5

through 8.

Input is free-format.  White space may appear anywhere in  a

program to enhance readability.

_#3.  _#I_#n_#t_#e_#r_#p_#r_#e_#t_#i_#n_#g _#t_#h_#e _#O_#u_#t_#p_#u_#t

Assuming that the _#P_#E_#G program for example 1  is  in  a  file

called  _#c_#o_#u_#n_#t_#e_#r,  the  following Unix command line may be used to

invoke _#P_#E_#G:

_#p_#e_#g _#c_#o_#u_#n_#t_#e_#r

The resulting output is shown in figure 3.  Generating a PLA from

the same input file is accomplished with the command line:

_#p_#e_#g _#c_#o_#u_#n_#t_#e_#r | _#e_#q_#n_#t_#o_#t_#t | _#m_#p_#l_#a -_#I -_#O

_#M_#p_#l_#a  will  not  automatically  connect  _#n_#e_#x_#t-_#s_#t_#a_#t_#e  outputs   to

_#c_#u_#r_#r_#e_#n_#t-_#s_#t_#a_#t_#e inputs.  After generating the PLA the state outputs

must be manually wired to the state inputs.

- 3 -

Designing Finite State Machines with PEG                  2/28/85

_#3._#1.  _#E_#q_#u_#a_#t_#i_#o_#n_#s

_#P_#E_#G generates the two  input  variables  _#I_#n_#S_#t_#0*  and  _#I_#n_#S_#t_#1*

INORDER          =          InSt0* InSt1*;

OUTORDER         =          OutSt1* OutSt0*;

OutSt1*          =          (!InSt1*);

OutSt0*          =          ( InSt0*&!InSt1*)| (!InSt0*& InSt1*);

Figure 3:  PLA Equations for Example 1.

which are the state inputs for the finite state machine.  It also

generates  two  output  variables  _#O_#u_#t_#S_#t_#0* and _#O_#u_#t_#S_#t_#1*, the next-

state outputs.  Any signal name ending with an asterisk was  gen-

erated by _#P_#E_#G.

The INORDER and OUTORDER statements specify that the result-

ing  PLA  inputs  and  outputs,  from  left to right, are InSt0*,

InSt1*, OutSt1*, and OutSt0*.

Following the OUTORDER statement are the logic equations for

the  two output variables, OutSt1* and OutSt0*.  The exclaimation

mark "!" indicates logical negation.  The ampersand "&" signifies

the  logical  _#A_#N_#D, while the vertical bar "|" signifies a logical

_#O_#R.

_#3._#2.  _#T_#r_#u_#t_#h _#T_#a_#b_#l_#e

The -t option generates a truth table for the  finite  state

machine.   This  truth  table is written to the file _#p_#e_#g._#s_#u_#m_#m_#a_#r_#y.

The truth table for example 1 is shown in figure 4.

INPUTS:                       s00:         InSt0* (msb)

s01:         InSt1* (lsb)

OUTPUTS:                      n01:         OutSt1* (lsb)

n00:         OutSt0* (msb)

State Table         s         s            n         n

0         1            1         0

0         0            1         0

0         1            0         1

1         0            1         1

1         1            0         0

Figure 4:  Truth Table for Example 1.

- 4 -

Designing Finite State Machines with PEG                  2/28/85

Labels across the  top  of  the  truth  table  identify  its

columns.   The  mapping from column labels to actual signal names

is given in the lists of input and output signals  which  preceed

the  truth  table.  To the right of the truth table are the names

of the states described by the rows of the table.

_#4.  _#A_#n_#o_#t_#h_#e_#r _#E_#x_#a_#m_#p_#l_#e

The second and more complex example shows the state  diagram

and corresponding _#P_#E_#G program for a FSM which recognizes the reg-

ular expression (1|0)*100.  The state diagram  for  this  FSM  is

shown in figure 5.

Figure 5:  State Diagram Recognizing (1|0)*100

The PEG program which implements this  design  is  given  in

figure  6.   Figure 6 describes a state machine with four states.

The state machine has two inputs, _#R_#E_#S_#E_#T and _#i_#n, and  one  output,

_#a_#c_#c_#e_#p_#t.

- 5 -

Designing Finite State Machines with PEG                  2/28/85

--Simple FSM example:  Accepts the regular expression (1|0)*100

INPUTS  :       RESET InStream;

OUTPUTS :       accept;

Top     :       IF NOT InStream THEN LOOP;            --0*

Saw1    :       IF InStream THEN LOOP;                --1

:       IF InStream THEN Saw1;                --10

:       ASSERT accept;

IF InStream THEN Saw1 ELSE Top;       --100

Figure 6:  PEG Program Recognizing (1|0)*100

INORDER           =          RESET InStream InSt0* InSt1*;

OUTORDER          =          OutSt1* OutSt0* Accept;

OutSt1*           =          (!RESET& InStream) |

(!RESET&!InStream& InSt0*&!InSt1*);

OutSt0*           =          (!RESET&!InStream& InSt0*&!InSt1*) |

!InStream&!InSt0*& InSt1*);

Accept            =          ( InSt0*& InSt1*);

Figure 7:  Equations for Example 2.

Assume the text of figure 2 is in a file called _#p_#r_#o_#g.  Logic

equations for the state machine are generated by running the com-

mand

_#p_#e_#g _#p_#r_#o_#g

Since this program has two inputs, they are declared in  the

_#I_#N_#P_#U_#T_#S  statement.   If a _#P_#E_#G program has any inputs they must be

declared in an _#I_#N_#P_#U_#T_#S statement which must be the first statement

in the program.  The input _#R_#E_#S_#E_#T is a special keyword input.  The

other program input, _#I_#n_#S_#t_#r_#e_#a_#m, is used to generate the _#n_#e_#x_#t _#s_#t_#a_#t_#e

for the FSM.

_#R_#E_#S_#E_#T indicates that when the _#R_#E_#S_#E_#T signal is  asserted  the

state machine jumps to the top of the program, which in this case

is named _#T_#o_#p.  When this keyword is present, conditional branches

to  the  first  state  are  automatically added to the _#n_#e_#x_#t _#s_#t_#a_#t_#e

- 6 -

Designing Finite State Machines with PEG                  2/28/85

INPUTS:              i00:  RESET

i01:  InStream

s00:  InSt0* (msb)

s01:  InSt1* (lsb)

OUTPUTS:             n01:  OutSt1* (lsb)

n00:  OutSt0* (msb)

o00:  Accept

State Table           i     i     s      s     n     n     o

0     1     0      1     1     0     0

1                -     0     0      0     0     -     Top

0                0     0     0      0     0     -     Top

0                1     0     0      1     0     -     Top

1                -     0     1      0     0     -     Saw1

0                0     0     1      0     1     -     Saw1

0                1     0     1      1     0     -     Saw1

1                -     1     0      0     0     -     Saw1+1

0                0     1     0      1     1     -     Saw1+1

0                1     1     0      1     0     -     Saw1+1

1                -     1     1      0     0     1     Saw1+2

0                0     1     1      0     0     1     Saw1+2

0                1     1     1      1     0     1     Saw1+2

Figure 8:  Truth table for Example 2.

expressions for each state.  If _#R_#E_#S_#E_#T is not listed as an  input,

the program initializes in a random state.

IF the FSM designer does not want to pay the  penalty  of  a

larger  and  slower finite state machine, _#R_#E_#S_#E_#T may be omitted as

it was in example 1.  In this case  the  reset  function  may  be

external  to  the  _#P_#E_#G  program by implementing the FSM in such a

manner that the _#n_#e_#x_#t _#s_#t_#a_#t_#e feedback lines are pulled low when the

_#R_#E_#S_#E_#T  signal is asserted.  This method will work because the top

state in a _#P_#E_#G program is always assigned to state zero.

The _#O_#U_#T_#P_#U_#T_#S statement declares that this program has a  sin-

gle  output called _#a_#c_#c_#e_#p_#t.  The FSM asserts this signal high if a

- 7 -

Designing Finite State Machines with PEG                  2/28/85

string in the given grammar is recognized.  If  any  outputs  are

generated  by  a _#P_#E_#G program, they must be declared in an _#O_#U_#T_#P_#U_#T_#S

statement which immediately follows the _#I_#N_#P_#U_#T_#S statement.  If  no

_#I_#N_#P_#U_#T_#S  statement  is  present, then the _#O_#U_#T_#P_#U_#T_#S statement is the

first program statement.

Example 2 introduces  the  _#I_#F-_#T_#H_#E_#N-_#E_#L_#S_#E  control  construct.

This  construct is used to provide two-way branches _#b_#a_#s_#e_#d _#o_#n_#l_#y _#o_#n

_#a _#s_#i_#n_#g_#l_#e _#i_#n_#p_#u_#t _#s_#i_#g_#n_#a_#l.  Branches based on  more  than  one  input

signal  are  handled by the _#C_#A_#S_#E statement which has not yet been

presented.  _#I_#F statements do not nest:  Statements  of  the  form

_#I_#F-_#T_#H_#E_#N-_#E_#L_#S_#E-_#I_#F are not allowed.  The syntax of the _#I_#F is:

_#I_#F [ _#N_#O_#T ] <_#s_#i_#g_#n_#a_#l> _#T_#H_#E_#N <_#s_#t_#a_#t_#e _#n_#a_#m_#e> [ _#E_#L_#S_#E <_#s_#t_#a_#t_#e _#n_#a_#m_#e> ] ;

The _#E_#L_#S_#E clause is optional:  If it is omitted, the _#E_#L_#S_#E defaults

to the next sequential state in the program.  Thus, in state _#T_#o_#p,

if _#I_#n_#S_#t_#r_#e_#a_#m is high, then the condition in the _#I_#F  is  false  and

the program takes the default branch to state _#S_#a_#w_#1.

The alert reader will have noticed that the state name  _#L_#O_#O_#P

is used but not defined.  This is intentional.  _#L_#O_#O_#P is a keyword

which means to stay in the current state.   It  is  an  error  to

define a state with the label _#L_#O_#O_#P.

The final state in example 2 shows  the  first  use  of  the

_#A_#S_#S_#E_#R_#T  statement.   The  _#a_#c_#c_#e_#p_#t  signal  is asserted only in the

accepting state of the FSM.  If an _#A_#S_#S_#E_#R_#T statement is present in

the  definition  of  a state, it must preceed the state's control

statement.

Figure 11 shows an ambiguous case specifier.  It is  ambigu-

ous because more than one case selector applies to the input (0 1

0).  In such cases _#P_#E_#G processes the list of case selectors  from

top  to  bottom,  using the first one that applies to the inputs.

Since the case specifier for State2 comes first, it  defines  the

next  state  for  inputs (0 1 0) and (0 1 1).  The case specifier

for State3 defines the next-state only for the case (1 1 0).

_#5.  _#F_#i_#n_#a_#l _#E_#x_#a_#m_#p_#l_#e

Figures 9 and 10 show the state diagram and _#P_#E_#G program  for

a  state  machine  which  decodes  3  bits  into  0, 1, 2, 3, and

"other".  Example 3 shows the use of  multiple  inputs,  multiple

outputs, and multi-way branches.

Multi-way branches and branches based on two or more  inputs

are  handled  by the _#C_#A_#S_#E statement.  The _#C_#A_#S_#E statement consists

of the keyword _#C_#A_#S_#E followed by an input signal list, a  list  of

case selectors, and an _#E_#N_#D_#C_#A_#S_#E.

- 8 -

Designing Finite State Machines with PEG                  2/28/85

Figure 9:  State Diagram for Example 3

A  case  selector  specifies  two  things:  a  bit   pattern

corresponding  to  the  input  signals, and a _#n_#e_#x_#t-_#s_#t_#a_#t_#e for that

combination of inputs.  Bit patterns are strings composed of  the

characters  '0',  '1',  and  signals  in  the  input signal list.

Don't-cares are specified with ?.

The  _#E_#N_#D_#C_#A_#S_#E  statement  optionally  specifies  the  default

next-state  if  none  of  the other case selectors applies to the

input.  In keeping with the model of sequential execution, if the

_#E_#N_#D_#C_#A_#S_#E does not specify a next-state, the next-state defaults to

the state following the one in which the _#C_#A_#S_#E statement appears.

- 9 -

Designing Finite State Machines with PEG                  2/28/85

--Decode inputs a, b, and c into

--0, 1, 2, 3, or "other".

INPUTS       :      RESET Select a b c;

OUTPUTS      :      Found0 Found1 Found2 Found3 FoundOther;

Start        :      --This is the reset state

IF NOT Select THEN LOOP;

:      CASE (a b c) --Second state

0 0 ? => Dummy; --A don't-care

0 1 0 => Two;

0 1 1 => Three;

ENDCASE=>Other;

Dummy        :      IF c THEN One;

Zero         :      ASSERT Found0; GOTO Start;

One          :      ASSERT Found1; GOTO Start;

Two          :      ASSERT Found2; GOTO Start;

Three        :      ASSERT Found3; GOTO Start;

Other        :      ASSERT FoundOther; GOTO Start;

Figure 10:  PEG Program for Example 3

State1             :             CASE (a b c)

0 1 ? => State2;

? 1 0 => State3;

ENDCASE=>State4;

Figure 11:  Ambiguity Resolution in Don't-Cares

- 10 -

Designing Finite State Machines with PEG                  2/28/85

_#6.  _#P_#e_#g _#S_#y_#n_#t_#a_#x

<program>       : <InputList> <OutputList> <StateList>

<InputList>     : INPUTS : <IdentList> ; | /*NULL*/

<OutputList>    : OUTPUTS : <IdentList> ; | /*NULL*/

<StateList>     : <State> | <StateList> <State>

<IdentList>     : <Identifier> | <IdentList> <Identifier>

<State>         : <Identifier> : <Signals> <Control> | : <Signals> <Control>

<Signals>       : /*null*/ | <ASSERT> <IdentList> ;

<Control>       : CASE ( <IdentList> ) <Cases> <DefaultCase>

| IF <Identifier> THEN <Identifier> ;

| IF <Identifier> THEN <Identifier> ELSE <Identifier> ;

| IF <NOT> <Identifier> THEN <Identifier> ;

| IF <NOT> <Identifier> THEN <Identifier> ELSE <Identifier> ;

| GOTO <Identifier>

| /*NULL*/

<Cases>         : <Cases> <CaseStmt> | <CaseStmt>

<CaseStmt>      : <BitList> => <Identifier> ;

<Bit>           : 0 | 1 | ?

<BitList>       : <BitList> <Bit> | <Bit>

<DefaultCase>   : ENDCASE => <Identifier> ; | ENDCASE ;

<NOT>           : "!" | "NOT" | "-"

<Comment>       : "--".*$

<Identifier>    : [A-Za-z][A-Za-z0-9._]*

- 11 -

Designing Finite State Machines with PEG                  2/28/85

_#7.  _#R_#e_#f_#e_#r_#e_#n_#c_#e_#s

[CADMan]

CAD Manual, Online Unix documentation.

[Danford]

Peggy Danford, Private communication with author, June 1982.

[Unix]

_#U_#n_#i_#x _#P_#r_#o_#g_#r_#a_#m_#m_#e_#r'_#s _#M_#a_#n_#u_#a_#l, _#4._#2  _#B_#e_#r_#k_#e_#l_#e_#y  _#S_#o_#f_#t_#w_#a_#r_#e  _#D_#i_#s_#t_#r_#i_#b_#u-

_#t_#i_#o_#n,  _#V_#i_#r_#t_#u_#a_#l  _#V_#A_#X-_#1_#1  _#V_#e_#r_#s_#i_#o_#n,  Computer Science Division,

University of California at Berkeley, November 1980.

- 12 -