.ce

.ps +2

.b "YACR User's Guide"

.ps -2

.ce

For Version 2.1

.sp 2

.ce

James Reed

.ce

Douglas Braun

.ls 1

.sh 1 "Problem Formulation" 1

.pp

The channels that YACR can route have the following characteristics:

.ip

i)   a rectangular region with no obstructions,

but possibly variations in the height of two opposite sides;

.br

ii)  two layers to be used for routing, referred to in this guide as "metal"

and "poly" but the actual material is unimportant;

.br

iii) fixed terminals on two opposite edges of the rectangular region,

called the "top" and "bottom" edges;

.br

iv) (optional) terminals on the other two edges, "left" and "right",

of the rectangular region,

exact position of these terminals is not specified,

but order may be specified for one edge, or exact locations

may be specified for one edge.

.pp

All of the routing is done within the boundaries of the rectangular

region.

The distance between the top and bottom edges is determined by YACR, but

will be minimized.

If fixed end terminals are specified, the width of the channel

is implicitly determined by the locations of the fixed end terminals.

The length of the top and bottom edges are given by the user, but in rare

cases may be extended by YACR to complete the route.

The top and bottom edges will be extended only by adding empty columns to

their left or right ends; the spacing between terminals on the top and

bottom edges will never be altered by YACR.

.pp

It is possible to specify variations in the height of the top and bottom edges.

This is done by specifying the relative distance towards the

center of the channel for each used or unused terminal position

on the top and bottom of the channel.

When this is done, the top or bottom of the channel can be thought of as

an irregular manhattan stairstep, with terminals only on the horizontal portions.

.pp

YACR is guaranteed to make all of the connections specified in its input,

unless fixed end terminals are specified,

in which case it will fail if the channel cannot be routed in the width

implicitly specified by the side with fixed end terminals.

The connections to the terminals on the top and bottom edges will be made

with poly.

The connections to the terminals on the left and right edges will be made

with metal.

.pp

YACR routes on a symbolic manhattan grid.

The metal layer is used for almost all routing segments that run parallel

to the top and bottom edges;  poly is used for almost all

segments parallel to the left and right edges.

This choice was made because the length of segments running from top

to bottom is usually less than the length of segments running from left

to right and metal has lower resistance than poly.

.pp

YACR is a

.i symbolic

channel router; it does not specify routing geometries in terms of actual

dimensions or location on a chip, but shows their relative positions in

the channel.

The output is comparable to a stick diagram of a circuit layout.

.pp

An interface to the HAWK graphics editor and SQUID data base that translates

the symbolic output of YACR into physical geometries has been developed.

For information on how this interface is used see

.i "User's Guide to Routing in HAWK" .

.ls 1

.sh 1 "Yacr Input and Output" 2

.sh 2 "Yacr Input File Formats"

.pp

Yacr can use either of two formats for input.

The information that is supplied by both formats

consists of:

the number of nets to be routed;

the number of columns in the channel;

a list of pins that connect to the top edge of the channel;

a list of pins that connect to the bottom edge of the channel;

the number of pins that connect to the left edge of the channel;

the list of pins on the left edge;

the number of pins that connect to the right edge of the channel; and

the list of pins on the right edge.

Two lists of "offsets" may also be specified.

The list of top offsets gives the relative distance towards the center

of the channel of each top pin.

The list of bottom offsets gives the relative distance towards the center

of the channel of each bottom pin.

First is a description of the default format,

then the format called for by the "-H" command line argument to Yacr.

.pp

The number of nets in the channel is specified with the following line:

.br

nnet= #

.br

The number must be separated from the equals sign by at least one white

space character, a space, a tab, or a newline.

No other spaces are allowed.

.pp

The number of columns in the channel is specified with the following line:

.br

ncol= #

.br

As with the number of nets, the only space allowed is between the equals

sign and the number.

The space is also required.

.pp

The list of pins on the top edge of the channel is begun with the keyword

"top_\|list".

The keyword is followed by a white space separated list of numbers,

representing nets.

The list must contain "ncol" integers.

The numbers need not be consecutive.

"0" is a special number that means "this location has no net connected to it."

.pp

The list of pins on the bottom edge of the channel is begun with the

keyword "bottom_\|list".

The restriction for the top list also apply to the bottom list.

.pp

The list of nets connecting to the left edge of the channel is begun

with the keyword "left_\|list".

The keyword is followed by a number telling how many nets there are

in the left list.

This is followed by the list of nets.

If there are no nets connecting to the left edge, this category may be

omitted.

By default, the order in which the nets are listed is not meaningful.

If a specific order is desired, "left_\|list" should be preceeded by the

keyword "relative".

The order in which the nets are listed is then the order (from top to bottom)

in which

they will appear in the final route.

If the word "fixed" instead of "relative", then an exact placement of

the edge nets is specified.

In this case, the edge list can contain zeroes like the top and bottom

lists.

The number of entries in a fixed edge list must be at least equal

to the inital width of the channel, which is the same as the maximum channel

density.

If the list is longer, the inital width of the channel will be increased

to the length of the list.

.pp

The list of nets connecting to the right edge of the channel is begun

with the keyword "right_\|list".

This category is exactly like the left list category.

An important restriction to note is that relative or fixed

order can be specified for only the left list

.U or

the right list.

If it is specified for both,

it will only be meaningful for the one that appears second in the

input file.

.pp

The optional list of offsets for the top pins of the channel is

begun with the keyword "top_\|offset".

It is followed by the list of offsets,

which must contain "ncol" integers.

Each entry gives the relative distance towards the center of

the channel of the corresponding pin location or channel boundary,

in grid units.

The offsets normally range from zero upwards (with nonzero

values representing an indentation into the side of the channel),

but the values are internally scaled, so only the relative values

matter.

If all the values are identical,

the top border of the channel is straight, and the results obtained

will be the same as if no offsets were specified.

.pp

The list of offsets for the bottom pins is begun

with the keyword "bottom_\|offsets", and

is similar to the list of top offsets.

Note that a positive value still means that the corresponding

pin or channel border is moved closer to

the center of the channel.

One or both of the offset lists may be omitted.

.pp

The "-H" format is identical to the default format except that all

keywords are omitted (except "relative" and "fixed") and

all categories must be

included in the following order:

.br

number of nets

.br

number of columns

.br

top list

.br

bottom list

.br

left list

.br

right list

.br

top offsets (optional)

.br

bottom offsets (optional)

.pp

The left list and the right list must be included, if there are no

nets connecting to the edge of the channel, the list will be

a single "0".

The file will contain nothing but integers separated by white space,

unless the keyword relative is included for the left or right list.

The top and bottom offsets may be omitted if the sides of the channel

have no indentations.

.sh 2 "Yacr Output File Formats"

.pp

Just as there are two input formats, there are two output formats.

One corresponds to the default input format and the other is used

with the "-H" flag or "-O" flag.

The default format is described first, then the differences between

it and the -H format follows.

.pp

The file begins with the following information:

input file; number of nets; number of columns; number of nets in the left

list; number of nets in the right list;

a list of edges removed from the vertical constraint graph to make it acyclic

(if any);

a list of columns with maximum density;

a list of columns that appear to be the best choices for starting column;

and the actual starting column.

.pp

The next section of the output contains a block for each "major attempt"

Yacr makes at routing the channel.

Each block has the number of rows used for the attempt,

followed by a list of vertical constraint violations (VCVs) that

occurred, followed by a list of how each VCV column was routed

(if it was).

If a row was added to complete the route without starting over,

that is included at the end of the last block.

.pp

The next and most important section gives the routing of the channel.

The horizontal (metal) layer is given first, followed by the

vertical (poly) layer.

Each layer is described by a matrix of integers.

Each row in the matrix represents a column in the channel

(this keeps the lines shorter so a printout is easier to read).

There are rows to represent the left and right edges of the channel;

the extra rows are used to indicate nets that connect to the edges.

Each integer in the matrices represents the net that occupies a given space

in the grid.

"0" means the space is empty.

If offsets were specified,

spaces that correspond to indentations in the channel

(where no material can be placed) are represented by "X".

Vias are not explicitly given, but implied at each location that

the metal layer has the same net as the poly layer.

.pp

Finally, there is a summary that tells the number of vias used in the route,

the total net length of all the nets, the longest net, and how much

metal and poly were used to route the longest net.

.pp

The "-H" output format contains a minimum amount of information.

It contains no keywords, just integers.

The first two lines have the number of rows and columns, respectively,

in the channel.

The next row is the top row of the metal layer of the channel, listed

from left to right.

The entire metal layer is given followed by the poly layer.

There are no extra columns representing the edges of the channel.

The last line of the file has three integers, the first is the

number of the net with the longest route, the amount of metal used

to route it, and the amount of poly used to route it.

Spaces inside indentations are now represented by a "0".

.sh 2 Examples

.ls 1

.sh 3 "Example 1, Input (Default Format)"

.nf

nnet= 2 ncol= 3

top_\|list

2 0 1

bottom_\|list

1 0 2

.sh 3 "Example 1, Output (Default Format)"

.nf

Input file: ex/2

num_\|nets= 2, num_\|cols= 3, num_\|left_\|nets= 0, num_\|right_\|nets= 0

Not closing cycle with edge from 1 to 2

Columns with maximum density:  1

Best choice(s) for starting column is (are):  1

initial_\|column = 1

num_\|rows = 2

VCV: nets  1  2, column   3

column 3 was not routed

Row 1 is being used to complete the route.

The final result is:

The horizontal layer (turned sideways):

0  0  0  col =   0

0  2  2  col =   1

0  2  0  col =   2

0  2  0  col =   3

0  0  0  col =   4

The vertical layer (turned sideways):

0  0  0   col =   0

1  0  2   col =   1

1  1  1   col =   2

2  2  1   col =   3

0  0  0   col =   4

There are 2 vias

The total net_\|length is 12

The longest net is 2, metal length = 4, poly length = 3

.sh 3 "Example 2, Input (Default Format)"

.nf

nnet= 72

ncol= 169

top_\|list

1  2  4  6  8 10 11 13  3  9 16  5 17 11  5 14 14  7 12 17 19  1 20 21

23 24  0 16 10  3 11 25  0 26 11 26 11  0 27 28 11  3  9 16 30 27  5 31

1  5  1 20 32 23 24  0  9  1 20 29 23 24  0  3  8 30 38 28 19  6 40 27

35 41 42  6 19 34 43 30  8 31 43 39 46 36 46 47 48 31  0 24 23 45 20  1

51  0 40 39 40 39  0  8 30 50 54  0  0 55 49 19  6  0 47 42 47 42  0 53

58  6 19 49 50 30  8 60 62 59 54 55 54 56 63 55 65  0 66 68 66 68  0 60

68  0 46 44 46 44  0 69  0 55 58 55 58  0 64 71  0 72 63 72 63  0 57 62

54

bottom_\|list

0  3  5  7  9  5 12 14 15  7 12 14  7  4 13  8  6 15 18 14  8  6 11 22

21  0 18 16 18 16  0  8  6 26 11  0 24 23 25 20  1 29  0 22  3 22  3  0

0  9  2  9  2  0 32 23 33 19  6  8 30 27 34 35 36 37 39 31 39 35 38 31

8 30 37 41 19  6 44 45  0 33 31 33 31  0 27 35 36 48 49 31 39 46 47 50

52 20 53 24  0 47 39  0 24 51 20 52 20 52 23  8 30 50 56  0  0 57 49 19

6  6 19 49 59  0  0 61 50 30  8 55  0 24 64 20 52  0 67 68 63 55 24 52

20 69 24  0 46 62 63 68  0 24 65 20 52  0 70 60 62 54 63  0 24 71 20 52

67

relative right_\|list 6

68 55 63 70 67 61

top_\|offsets

0  0  0  0  1  1  1  3  3  3  3  3  3  3  3  3  3  3  2  2  2  2  2  2

2  2  2  2  2  2  2  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  1  1

0  0  0  0  1  1  1  3  3  3  3  3  3  3  3  3  3  3  2  2  2  2  2  2

0  0  0  0  1  1  1  3  3  3  3  3  3  3  3  3  3  3  2  2  2  2  2  2

2  2  2  2  2  2  2  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  1  1

1  1  1  1  5  5  5  5  5  5  5  5  5  4  4  4  4  4  4  4  4  4  4  4

2  2  2  2  2  2  2  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  1  1

1

bottom_\|offsets

2  2  2  2  2  2  2  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  1  1

0  0  0  0  1  1  1  3  3  3  3  3  3  3  3  3  3  3  2  2  2  2  2  2

0  0  0  0  1  1  1  3  3  3  3  3  3  3  3  3  3  3  2  2  2  2  2  2

0  0  0  0  1  1  1  3  3  3  3  3  3  3  3  3  3  3  2  2  2  2  2  2

2  2  2  2  2  2  2  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  1  1

2  2  2  2  2  2  2  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  1  1

1  1  1  1  5  5  5  5  5  5  5  5  5  4  4  4  4  4  4  4  4  4  4  4

4

.ls 1

.br

.sh 1 "Running YACR" 3

.sh 2 Description

.pp

YACR2 is run with the following command:

.ce

yacr [options] [file1 [file2]\|]

.br

File1 contains the input, file2 gets the output.

If file2 is omitted, the output goes to standard out.

If no files are specified, input is read from standard input,

and output goes to standard output.

.pp

The command line options described below can be specified in any

order, but must come before the input and output file names.

.ip -a

Add columns at the ends of the channel if necessary to complete

the route.

By default columns are not added,

but if the channel has cyclic constraints, additional columns

may be necessary.

If

.i yacr

cannot complete a route, it will give a message recommending

that this flag be used.

.ip "-c n"

Begin routing the channel starting in column "n".

If this is not specified,

or if "n" is less than one or greater than the number of columns

in the channel,

.i yacr

will chose a starting column.

If the left_\|list or right_\|list is specified in "relative"

or "fixed" order,

the "-c" option will be ignored.

.ip -d

Sets the debug flag so that the channel will be printed

(in the desired format)

at the following times:

when the current number of rows is found to be insufficient;

when the route is completed, before metal maximization and cleanup;

after metal maximization and cleanup.

Useful only for debugging purposes.

.ip -H

Use an alternate input and output format.

Also forces input to be read from standard input,

and output to be written to standard output.

.ip "-O filename"

Read the normal input format, but place the results in the

alternate format in "filename".

The statistics, etc., are still written to any output

file specified in the normal way.

Useful to prepare output for arrayToCif or other post-processors.

.ip -s

Do not generate output for the channel;

only print statistics for the channel width, number of vias, etc.

.ip -C

Do not print the list of cycles in the vertical constraint graph.

Channels with many fixed end pins may have thousands of cycles.

.ip -m

Do not perform metal maximization.

.sh 2 Diagnostics

.pp

The following error messages are all written to stderr.

Yacr will return a nonzero exit status if there is any fatal error.

.sp

.nf

"net n is not properly routed

bad pins are: col1/edge1 .\|.\|.\|"

.fi

The verification routine has discovered a problem with the route.

The pin of net "n" in column "col1" on edge "edge1" was not connected

to the leftmost pin of net "n".

This message will only appear when a bug in the code manifests itself.

.pp

.nf

"The channel cannot be routed without additional columns,

please use the "-a" command line argument."

.fi

The channel has cyclic constraints that cannot be routed in the

space provided.

This has not happened on any of the hundreds of industrial channels

routed by YACR to date.

.pp

.nf

"top_\|list already specified, first list used."

.fi

The input file tries to specify the top_\|list portion of the input

more than once, all lists after the first are ignored.

There are similar messages for the bottom_\|list, left_\|list,

right_\|list, nnet, and ncol.

.sh 2 "Suggestions for Best Results"

.pp

The best suggestion that can be given here is:  Let YACR make as many

decisions as possible.

There are two ways in which the user may make decisions for YACR, either

by specifying that the left_\|list or right_\|list is "relative" or

"fixed", or by

specifying the starting column with the "-c" command line argument.

One decision that the user should make for YACR is whether or not it

should be allowed to add columns to the channel ends to complete the route.

.uh "Relative Order"

.pp

If the left_\|list or right_\|list is given a specfic order YACR will almost

always require more rows for routing that if the order was not given.

This happens for two reasons:  first, there are more restrictions that must

be met by the final route; and second, in order to guarantee the restrictions

are met, YACR must begin routing at either the left or right end of the

channel (usually a bad place to start).

If a fixed left or right list is given, there will be

even more restrictions.

Also, if there are fixed end pins,

the channel width will not be automatically expanded

if Yacr fails to route. In this case, it is

necessary to restart the problem, with another row

specified by placing it in the list of fixed end pins.

Remember that the list of fixed end pins must be at

least as large as density.

.uh "Use of -c"

.pp

Most of the time that YACR is allowed to choose its starting column

it will route a channel in density.

Occasionally, a small number of extra rows will be required to complete the

route.

Since the number of rows needed for the route will vary by one or two

depending on which column YACR started with, the user can sometimes

get YACR to route a channel in density by forcing it to begin in a column

it would not normally choose.

.pp

Note that if the pins on the left or right edge have a specified order,

the user is already forcing YACR to start in the first or last column.

.pp

The YACR output file contains information that is useful in picking a

starting column that might be better than the column YACR chose.

Consider the following portion of a YACR output file (only the routing

of the channel is omitted):

.nf

.in +.5i

.ls 1

Input file: ex/3c

num_\|nets= 54, num_\|cols= 103, num_\|left_\|nets= 0, num_\|right_\|nets= 0

Columns with maximum density:  58 69

Best choice(s) for starting column is (are):  58 69

initial_\|column = 58

num_\|rows = 18

VCV: nets 38 45, column  71

VCV: nets 21 45, column  63

VCV: nets 21 24, column  75

column 75 was routed by maze1a

column 63 was not routed

column 71 was not routed

Row 1 is being used to complete the route.

.in -.5i

.sp

.ls 1

.fi

In this example YACR was allowed to choose a starting column.

It chose column 58, and ended up with two

vertical constraint violations (VCVs)

(in columns 63 and 71) that it could not remove.

We could hope for better results by forcing YACR to start in any of

columns 69, 63, or 71.

We might choose 69 because YACR said it might be a good choice, or

we might choose 63 or 71 in hopes that a difficult-to-remove

VCV would be avoided.

If we choose to force YACR to start routing in column 63 (with the

command "yacr -c 63 inputfile outputfile") we get the following results.

.nf

.in +.5i

.ls 1

Input file: ex/3c

num_\|nets= 54, num_\|cols= 103, num_\|left_\|nets= 0, num_\|right_\|nets= 0

Columns with maximum density:  58 59 69 70

initial_\|column = 63

num_\|rows = 18

VCV: nets 41 26, column  72

VCV: nets 49 35, column  90

column 72 was routed by maze2

column 90 was routed by maze2

.sp

.ls 1

.in -.5i

.fi

With the forced starting column YACR was able to route the channel in density.

.pp

There are several interesting things should be pointed out about the two

output files.

Notice that the first one lists 58 and 69 as the columns with maximum density

and the second lists colums 58, 59, 69, and 70.

This is not a contradiction, in this example columns 58 and 59 are

crossed by the same nets (the same is true of columns 69 and 70).

If YACR began routing in either column 58 or 59, it would achieve exactly

the same results, so it removes all but one of the redundant columns from

consideration.

When the user specifies a starting column, YACR does not waste time

figuring out if adjacent columns of maximum density are identical.

.pp

The other interesting fact is that there were not only a different number

of columns with vertical constraint violations, but the columns were

totally different when YACR began routing in a different column.

.uh "Irregular Channels"

.pp

Yacr can handle channels with irregular sides.

This is done by specifying the top and bottom offset lists

in the input.

Some points must be remembered.

First, the offsets apply to the places where there are no pins,

as well as the columns with pins.

Second, if fixed end pins are specified at the same time as

irregular edges, the horizontal segments of the fixed pins

must not run into the indentations.

Yacr will fail if this happens.

Last,

Yacr was only designed to handle relatively small indentations.

If the effect of the indentations is to give the channel

a zig-zag shape, and there are nets that span a large portion

of the channel, Yacr will use more tracks than necessary.

This is because Yacr does not dogleg nets, so the nets

cannot bend to get around the indentations.

.uh "Use of -a"

.pp

There are some cases in which YACR will think it needs extra columns, but

could actually complete the route by adding only rows.

Since the times when YACR really needs extra rows are rare (it has not yet

happend in a industrial example), you should only use the "-a" command

line argument after YACR tells you it is necessary.

.sh 2 "Reed's Rule of Routing"

.pp

Routers do weird things.

.pp

When looking at the output of YACR (or any other router) it

is always a good idea to keep in mind

this variant of the widely known Murphy's law.