/*

* Revision Control Information

*

* $Source: /users/pchong/CVS/sis/sis/map/library.doc,v $

* $Author: pchong $

* $Revision: 1.1.1.1 $

* $Date: 2004/02/07 10:14:25 $

*

*/

/* file @(#)library.doc	1.3 */

/* last modified on 9/16/91 at 17:08:24 */

A library consists of a number of gates.  All of the gates which have

the same logic functions are grouped into 'classes'.  The basic data

types are library_t, lib_class_t, and lib_gate_t.  It is verboten to

look inside these structures, because the organization of the data is

still being refined.

library_t *

lib_get_library()

Returns the 'current' library, or NIL(library_t) if a current

library has not been defined.

lsGen

lib_gen_classes(library)

library_t *library;

Initialize an lsGen generator for the class in a given library.

See generator usage example below.

lsGen

lib_gen_gates(class)

lib_class_t *class;

Initialize an lsGen generator for the gates in a given class.

See generator usage example below.

lib_class_t *

lib_get_class(network, library)

network_t *network;

library_t *library;

(For backward compatibility)

Given a 'network', find the COMBINATIONAL gate-class which has the same logic

function.  Use lib_get_class_by_type for find other gate-classes.

Returns NIL(lib_class_t) if there is no such gate in

the library.  A fast way to construct a network is using

read_eqn_string().

lib_gate_t *

lib_get_gate(library, name)

library_t *library;

char *name;

Get a gate by its name.  Returns NIL(lib_gate_t) if no gate with

that name is found in 'library'.

/*

*  Attributes of a class

*/

char *

lib_class_name(class)

lib_class_t *class;

Returns the name of a class.  Currently, a class is named by the

name of its first gate (essentially, a randomly chosen gate).

lib_class_t *

lib_class_dual(class)

lib_class_t *class;

Returns the dual class of a class, or NIL(lib_class_t) if the

given class does not have any dual gates in the library.

network_t *

lib_class_network(class)

lib_class_t *class;

Return a 'network' which represents the logic function associated

with a class.  Do not alter or free this network.

/*

*  Attributes of a gate

*/

char *

lib_gate_name(gate)

lib_gate_t *gate;

Returns the name of a gate.  Do not free or alter the return string.

char *

lib_gate_pin_name(gate, pin, inflag)

lib_gate_t *gate;

int pin;

int inflag;

Returns the name of a pin on a gate.  Do not free or alter the

return string.  Returns NIL(char) if 'pin' is more than the

number of pins on the gate.  If 'inflag' is 1, then the n'th

input pin name is returned; if 0, then the n'th output pin name

is returned.  If 'inflag' is 2, then the control (clock) pin

name is returned ('pin' is ignored in this case).

Pin numbers start at 0.

double

lib_gate_area(gate)

lib_gate_t *gate;

Return the area of a gate.

lib_class_t *

lib_gate_class(gate)

lib_gate_t *gate;

Returns the class of a gate.

int

lib_gate_num_in(gate)

lib_gate_t *gate;

Returns the number of inputs of the gate.

int

lib_gate_num_out(gate)

lib_gate_t *gate;

Returns the number of outputs of the gate.

/*

*  Mixing the library and node packages

*/

lib_gate_t *

lib_gate_of(node)

node_t *node;

Return the gate which is currently implementing 'node'.  Returns

NIL(lib_gate_t) if this node is not mapped.  Note that PRIMARY_INPUT

and PRIMARY_OUTPUT nodes are never mapped; hence, lib_gate_of()

will always return NIL(char) for them.

int

lib_network_is_mapped(network)

network_t *network;

Returns 1 if the network is mapped (i.e., all nodes in the network

have an associated gate from a library), or 0 if not.

int

lib_set_gate(node, gate, formals, actuals, nin)

node_t *node;

lib_gate_t *gate;

char **formals;

node_t **actuals;

int nin;

This can be used to either (1) set the implementation

of the 'node' to be 'gate' or (2) create a latch

with the 'node' as the latch output and 'actuals' as

input pins to the latch.   Usage (2) is assumed

if 'node' is a primary input,(1) otherwise.

(1) This sets the implementation of the given node to be 'gate'.

This changes the logic function of the 'node', and sets the node as

'mapped'.   'formals' is an array of names of the formal terminals of the gate

(i.e., the gate pin names).   'actuals' is an array of nodes in

the network (should be the same network that 'node' belongs to)

which is matched in 1-1 correspondance 	with 'formals'.

As a consistency check, 'nin' declares the

sizes of the arrays 'formals', and 'actuals'.  'nin' should be

equal to the number of primary inputs of gate->network.

Returns 1 on success, 0 if any error.  Probable errors are:  gate is

NIL(lib_gate_t), number of inputs declared via 'nin' does not

match the number of inputs on 'gate', the specified gate is not

single-output, or some name in 'formals' is not a pin on the

gate.  error_string() is appended to in the case of an error.

(2) This creates a latch connection with the 'node' as the latch

output and an internally created primary output as the latch input.

An internal node is also created and its implementation is

set to be 'gate'.   'formals' and 'actuals' are the same as

those defined in (1) except that they do not include latch output

pins.  'nin' in this case is equal to network_num_pi(gate->network) - 1

for complex latches (latches with more than one input) and

1 for D-type latches/flip-flops.   The following fields of the newly

created latch structure (latch_t) are set: input, output, synch_type and gate.

The rest of the fields (initial value, current_value and control) must be

set by the calling program.

Generator usage example:

lsGen gen, gen1;

library_t *library;

lib_class_t *class;

lib_gate_t *gate; char *dummy;

library = lib_get_library();

gen1 = lib_gen_classes(library);

while (lsNext(gen1, &dummy, LS_NH) == LS_OK) {

class = (lib_class_t *) dummy;

/*

*  do something with class

*/

gen2 = lib_gen_gates(class);

while (lsNext(gen2, &dummy, LS_NH) == LS_OK) {

gate = (lib_class_t *) dummy;

/*

*  do something with gate

*/

}

(void) lsFinish(gen2);

}

(void) lsFinish(gen1);

/* sequential support */

latch_synch_t

lib_gate_type(gate)

lib_gate_t *gate;

Given a library gate, return the type of the gate (i.e., COMBINATIONAL,

ACTIVE_HIGH, ACTIVE_LOW, RISING_EDGE, FALLING_EDGE, ASYNCH or UNKNOWN).

int

lib_gate_latch_pin(gate)

lib_gate_t *gate;

Given a latch/flip-flop, return the fanin index corresponding to

the latch output pin.   If no such fanin exists (combinational

gates and D-type latches/flip-flops) return -1.

latch_time_t**

lib_gate_latch_time(gate)

lib_gate_t *gate;

Given a latch/flip-flop, return the timing information (setup/hold)

associated with the latch.

delay_pin_t*

lib_gate_clock_delay(gate)

lib_gate_t *gate;

Given a latch/flip-flop, return the pin delay from the clock pin to the

output pin.

lib_class_t *

lib_get_class_by_type(network, library, type)

network_t *network;

library_t *library;

latch_synch_t type;

Given a 'network', find the gate-class of appropriate type which

has the same logic function.  Returns NIL(lib_class_t) if there is

no such gate in the library.  A fast way to construct a network is using

read_eqn_string().

lib_gate_t *

lib_choose_smallest_latch(library, string, type)

library_t *library;

char *string;

latch_synch_type type;

Given an equation string, return the smallest combinational

gate or latch of appropriate type which has the logic function

specified in the string.   For example,	to find a smallest D-type

transparent latch which is active high,	use

lib_choose_smallest_latch(library, "q=d;" ACTIVE_HIGH).