NET -- simple network description language

SYNOPSIS

net infile [outfile] [-o] [-s#] [-d#,#] [-e#,#] [-n]

DESCRIPTION

NET provides a simple language for describing MOS circuits (include BiCMOS);

the language includes macros, iteration, conditionals, etc.  The NET

program expands a description prepared by the user into a list of

transistors (a "netlist" in the .sim format).

infile is the name of the net input file, if outfile is specified, that file is

used for output.  The options are:

-n	disables error report about use of undeclared name.

-o	old format input.  size specifications are taken to be length/width

rather than width/length.

-s#	use specified number as initializer for internal node names; useful

when you want to merge the results of separate NET runs.

-d#,#	set the default width (first number) and length (second number) for

depletion devices.  The defaults are 8 and 2.

-e#,#	like -d except for e-cap (poly-poly2 capacitor) devices.  The

defaults are 2 and 2.

-i#,#	like -d except for intrinsic devices.  The defaults are 2 and 2.

-l#,#	like -d except for low-power devices.  The defaults are 2 and 2.

-p#,#	like -d except for p-channel devices.  The defaults are 2 and 2.

A NET file can insert other net files by using the include command:

(include "filename.net")

The single argument must be a string (i.e., enclosed in quotes).

Available through NET are all the regular built-in functions of RNL (i.e.,

a subset of standard LISP primitivies) -- see rnl.doc for a description of

these subroutines.  In addition, NET offers some special functions useful

for building a description of a transistor network.  These functions are

described below.

NET is a macro-based language for describing networks of sized transistors.

Names in NET refer to nodes, which presumably get interconnected by the user

through transistors.  A node name has two forms

(n width length)	n is the name of the network node, length

and width specify a transistor size -- this

form is used in NET constructs where mention

of a name causes creation of a transistor...

n			n is the name of the network node; when

transistor sizes are required they are taken

from the appropriate defaults

When using a name to refer to a node, it must first be "declared" (this

allows typos to be caught early on).  Nodes are declared by using the

node statement or the local statement (see below).  The node statement

looks like:

(node n1 n2 n3 ...)

where n1, n2, etc are the names to be declared.  Note: when using structured

names (see the repeat statement) only the first component has to be declared.

The interconnect capacitance associated with a node can be specified as

follows:

(capacitance n 1.234)

(setq pf-sq-micron-of-diffusion 10e-4)

(capacitance n (* 13 pf-sq-micron-of-diffusion))

The first argument is the name of the node, the second the capacitance in

pf (must be a number).

An electrical node can be given several names by using the connect statement:

(connect n1 n2 n3 ...)

The names n1, n2, etc. will all refer to the same electrical node.  This

statement is useful for connecting i/o signals to the edge of an array

generated by a repeat statement.

The voltage threshold for logic high and low states can be set by the

NET command threshold:

(threshold n 0.2 0.8)

would set the logic low threshold for node n to 0.2 (normalized voltage)

and the high threshold to 0.8.  If no threshold is specified, the node

will be given the default thresholds as given in the configuration file

for PRESIM (see PRESIM.DOC for details).

The "delay" of a node (the transition times for changes in the node's

value) can be specified by user with the delay command:

(delay n plh phl)

where plh and phl are integers specifying the low-to-high transition

delay and the high-to-low delay respectively.  Delays are specified

in RNL time units (1/10th nanosecond).  If you do not specify a delay

for a node, RNL will calculate one based on the impedence of the driving

transistors and the capacitance of the node; user-specified delays

override the usual RNL calculation.

Node interconnections are accomplished by one of the following NET

statements:

(trans g s d [w [l]])

(etrans g s d [w [l]])	enhancement mode transistor with gate g,

source s, and drain d.  l and w specify length

and width of transistor (can be ommited).

Also used in a kludgy manner to match magic's

ext2sim output for poly-poly2 caps, where g

(gate) is poly2 and s,d (source, drain) are

the same node (poly).

(dtrans g s d [w [l]])	like etrans, except depletion mode transistor

(itrans g s d [w [l]])	like etrans, except intrinsic transistor

(ltrans g s d [w [l]])	like etrans, except low-power transistor

(ptrans g s d [w [l]])	like etrans, except p-channel transistor

(btrans b e c [w l])		a vertical bipolar npn transistor in an

analog n-well BiCMOS process, with base

b, emitter e, and collector c.  The optional

width and length (w,l) define the area of

the nwell containing the transistor, and

are permutable.

(resistor a b resistance)	resistor of specified size between nodes a

and b.

Iteration construct is repeat statement:

(repeat index low high

[(local l-1 ... l-j)]

...)

where index will be given successive values starting with low and

finishing with high.  You can use the index in structured names, e.g.:

foo.index   foo.(1+ index)  foo.(1- index).bar  ...

local variables are described under macros.

For ease of circuit entry, the user can build and call parameterized macros.

macro definitions have the form

(macro n (p-1 ... p-k)

[(local l-1 ... l-j)]

...)

where n is the name of the new NET function being created

p-1 ... p-k are the formal parameters

l-1 ... l-j are the optional local node names used in the body

The macro is invoked as follows:

(n a-1 ... a-k)

which causes the body to be interpreted after

(1) all occurrences of p-1 in the body have been replaced by a-1, etc.

(2) all occurrences of l-1 in the body have been replaced by a new, unique

node name.  Unique names will be a number (like for anonymous nodes

in pulldowns).

In the following examples

e g s d l w

specifies an enhancement-mode transistor with gate g, source s, and drain d

with length l and width w.

d g s d l w

is similiar, except transistor is depletion mode.

Quickie examples:

(invert a b)

d a a vdd 8 2

e b a gnd 2 2

(invert a (b 17 5))

d a a vdd 8 2

e b a gnd 5 17

(invert (a 2 2) (b 2 4))

d a a vdd 2 2

e b a gnd 2 4

(nor (a 16 2) (b 2 4) c d)

d a a vdd 2 16

e b a gnd 4 2

e c a gnd 2 2

e d a gnd 2 2

(and-or-invert a (b c d) (e f) (g))

d a a vdd 8 2

e b a 1001 2 2

e c 1001 1002 2 2

e d 1002 gnd 2 2

e e a 1003 2 2

e f 1003 gnd 2 2

e g a gnd 2 2

Two dimensional array of foo's:

(repeat i 1 8 (repeat j 1 8 foo.i.j))

generates

foo.1.1   foo.1.2   foo.1.3   ...  foo.1.8

foo.2.1   ...		       foo.8.8

Simple two-inverter dynamic memory cell:

(macro bitcell (output output-enb input input-enb refresh)

(local a b c)

(trans input-enb input a 2 4)

(invert b a)

(invert (c 2 2) (b 2 8))

(trans refresh a c)

(trans output-enb c output 2 4)

)

(bitcell bit0 renb bit0 wenb phi2)

generates

e wenb bit0 1001 4 2

d 1002 1002 vdd 8 2

e 1001 1002 gnd 2 2

d 1003 1003 vdd 2 2

e 1002 1003 gnd 8 2

e phi2 1001 1003 2 2

e renb 1003 bit0 4 2

Assume you had an alu bit-slice macro of the following form

(alu carry-in operand1 operand2 result carry-out)

then the following macro would produce an n-bit alu:

(macro ALU (n databus1 databus2 resultbus cin cout)

(connect cin cout.0)

(repeat i 1 n

(alu cout.(1- i) databus1.i databus2.i resultbus.i cout.i))

(connect cout cout.n)

)

Instead of using the connect statement one could have conditionalized

the calculation of the arguments to alu:

(macro ALU (n databus1 databus2 resultbus cin cout)

(repeat i 1 n

(alu (cond ((== i 1) cin) (t cout.(1- i)))

databus1.i

databus2.i

resultbus.i

(cond ((== i n) cout) (t cout.n))) ))

)